3GPP TSG-SA3 Meeting #117	S3-243309

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Ericsson

# Title:	New Solution to Key Issue #1: Using security log events, counters and protocol signaling monitoring

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Approve the pCR to TR 33.794 [1] below.

# 2	References

[1]	3GPP TR 33.794 "Study on enablers for Zero Trust Security"

# 3	Rationale

This contribution proposes a new solution to Key Issue #1 of TR 33.794 [1].

# 4	Detailed proposal

*** BEGIN CHANGES ***

## 7.Y	Solution #Y: Using security log events, counters and protocol signaling monitoring

### 7.Y.1	Introduction

This solution addresses Key Issue #1: Data exposure for security evaluation and monitoring, aspect (1) "Specification of data (stage-2) to be collected for security evaluation and monitoring of the 5G SBA".

The solution describes how security log events, counters and protocol signaling monitoring tools can be used to address the use cases in clause 5.1 of the present document.

### 7.Y.2	Solution details

#### 7.Y.2.1	General

For the operation and management of mobile networks, different data sources can be used to understand the status of the network which in turn allows to take measures based on the understanding. Some of the data sources are specified in 3GPP documents, but most are not. For security monitoring, examples for usual data sources today are security log events, different counters and protocol signaling monitoring tools. Since security logs and counters are usually proprietary, every implementation will have slightly different types of events and counters. However, many are quite straightforward, and it seems reasonable to assume that some version of them exists in most implementations.

In the following clauses, it is described how security log events, counters and protocol signaling monitoring tools can be used to address the use cases in clause 5.1. Since security log events and counters are usually proprietary, normative work based on this solution could be on the requirement level.

#### 7.Y.2.2	Use case #1: Information on Malformed Message

It is common security practice to drop malformed messages, and also common practice to perform fuzz testing on interfaces to check that malformed messages indeed are dropped. As explained in clause 5.1.1.1, besides dropping the malformed message, it could also be desirable to monitor the occurrence of malformed messages. However, sending events for every malformed message could also introduce an opportunity for a denial-of-service attack on the monitoring system. Hence, for the use case of malformed messages, it seems preferable to use counters for the number of malformed messages that are dropped. Once the counter reaches a certain configurable threshold, the security monitoring system could be informed.

#### 7.Y.2.3	Use case #2: Massive number of SBI Messages

Handling overload scenarios is a common task for mobile networks, mostly for operational reasons since a high load can also occur for many benign reasons. However, as explained in clause 5.1.2.1, it could also be desirable to monitor the signalling load for security reasons. Especially for this use case, it is important that a high load and/or denial of service attack on the SBA signalling layer does not also lead to a high load or denial of service at the security monitoring layer. Hence it seems a better choice to use counters and thresholds to inform the security monitoring layer about an unusual high load.

#### 7.Y.2.4	Use case #3:  Unauthorized/failed authentication NF service access request

If an unauthenticated or unauthorized NF tries to invoke a service at another NF or at the NRF, the service will not be provided. As explained in clause 5.1.3.1, it could be desirable to monitor the events of attempted service access without authentication or authorization. Such unauthenticated or unauthorized service access could result in a security log event that is exported to the security monitoring function. Again, care needs to be taken so that this does not introduce an opportunity for a denial-of-service attack on the security monitoring system.

#### 7.Y.2.5	Use case #4:  Reconnaissance

Use case #4 describes scenarios where a valid TLS connection is set up but not used in the intended way, e.g. no API call is made. In these scenarios, there is no actual error, but instead a valid communication event is interpreted as an indication of a potential attack.

The monitoring of valid communication can be done by protocol signaling monitoring tools, which is a usual practice in network operations and management.

Use case #4 also describes the scenario where a TLS connection is attempted but not fully established. However, this type of event depends highly on the TLS stack implementation and is potentially not exported from the TLS stack.

#### 7.Y.2.6	Use case #5: Abnormal SBI Call Flow

Use case #5 describes scenarios where service requests are authenticated and authorized but appear outside valid communication flows and procedures. Such requests are usually caught by the internal logic of the NF and the messages are silently discarded. As explained in clause 5.1.5.1, it could be desirable to also monitor the occurrence of such discarded messages. This could be done by counters, in order to not impact performance and introduce the opportunity for a denial-of-service attack on the security monitoring system. Alternatively, protocol signaling monitoring tools could used for this use case as well.

#### 7.Y.2.7	Use case #6: API Security Risks

Use case #6 describes scenarios where service requests are authenticated and authorized but are duplicates, out of sequence or exceed the limits for e.g. message length.  Such requests are usually caught by the internal logic of the NF and the messages are silently discarded. As explained in clause 5.1.6.1, it could be desirable to also monitor the occurrence of such discarded messages. This could be done by counters, in order to not impact performance and introduce the opportunity for a denial-of-service attack on the security monitoring system. Alternatively, protocol signaling monitoring tools could used for this use case as well.

### 7.Y.3	Evaluation

This solution proposes to use security log events, counters and protocol signaling monitoring tools to address the use cases in clause 5.1. This is well aligned with existing practice and allows to perform security monitoring following the use cases with minimal deployment effort and implementation impact. Since security log events and counters are usually proprietary, normative work based on this solution could be on the requirement level.

*** END CHANGES ***