3GPP TSG-SA3 Meeting #117	S3-243320

Maastricht, Netherlands  19 - 23 August 2024

Title:	Reply LS on FS_5GSAT_Ph3_ARCH conclusions

Response to:	LS (S2-2407350) on FS_5GSAT_Ph3_ARCH conclusions from SA2

Release:	Rel-19

Work Item:	FS_5GSAT_Ph3_ARCH

Source:	Qualcomm Incorporated (to be SA3)

To:	SA2

Cc:	SA3-LI, RAN2

Contact person:	Adrian Escott

aescott@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 thank SA2 for their LS (S3-2407350/S3-242725) on FS_5GSAT_Ph3_ARCH conclusions from SA2.

In that LS, SA2 asks SA3 the following question:

SA2 kindly asks SA3 to take the TR 23.700-29 Clause 8.2 and Clause 8.3 into account and provide feedback, if any.

For clause 8.2, SA3 has not found any serious security concerns provided certain restrictions are supported.

For a full EPC including HSS in a satellite, UE security credentials that are exposed in a satellite must not include credentials in a USIM and should be updated periodically and not reused. This is possible using an IOPS based solution.

For the split MME solution with HSS on the ground, the next satellite that the UE accesses must have and be the only entity that has the UE’s current NAS security context in order to avoid NAS security problems that can occur when a UE attempts to access a satellite with a prior NAS security state.

# 2	Actions

To SA2

ACTION: 	SA3 asks SA2 to take the above response into account in their further work.

# 3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 October 2024		Hyderabad (India)

SA3#119	11 - 15 November 2024		Orlando (US)