3GPP TSG-SA3 Meeting #117	S3-243338

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	Qualcomm Incorporated

# Title:	Updates on solution #14

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes to update the solution #14.

# 2	References

[1]	TR 33.743

# 3	Rationale

This contribution proposes to update the solution #14.

Particularly, it is proposed to remove two Editor’s Notes and add a solution evaluation.

First, End UE User Info is associated to an RSC and it is only shared to 5G ProSe UE-to-UE relays that are authorized to provide the multi-hop relay service associated with the RSC. This means an attacker that is not authorized to provide the relay service associated with the RSC cannot obtain an End UE User info specific to an attacker’s target End UE. Thus, the privacy of End UE against unauthorized attackers are preserved during relay discovery in this solution. Therefore, it is proposed to remove the following Editor’s Note.

Editor’s Note: How is privacy of End UE during discovery preserved when Relays share End UE User info (with associated IP address) to other Relays when using MANET based routing discovery is FFS.

Second, this solution is aligned with the SA2’s conclusion, especially when a multi-hop UE-to-UE relays with PDU type IP is used. Thus, it is proposed to add a clarification that this solution applies to the PDU type IP in the evaluation and remove the following Editor’s Note.

Editor’s Note: Alignment with SA2’s conclusion about the procedure is needed.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [1].

**** START OF CHANGES ****

## 6.14	Solution #14: Multi-hop UE-to-UE Relay discovery security

### 6.14.1	Introduction

This solution addresses the first, third and fourth security requirements in the key issue #2 regarding the multi-hop UE-to-UE (U2U) Relay discovery. This solution assumes the architecture and procedures proposed in the solution #3 of TR 23.700-03 [1]. This means that 5G ProSe UE-to-UE Relays discover each other to form a 5G ProSe UE-to-UE Relay cloud, and 5G ProSe End UEs first discover nearby 5G ProSe UE-to-UE Relay and discover a target 5G ProSe End UE at IP layer (e.g., based on MANET routing protocol) via 5G ProSe UE-to-UE Relay cloud.

This solution proposes to reuse the security procedure for 5G ProSe UE-to-Network Relay discovery with Model A and Model B as specified in clause 6.1.3.2.2 of TS 33.503 [5]. That is, the discovery messages are protected based on the discovery security materials associated with an RSC for multi-hop UE-to-UE Relay.

### 6.14.2	Solution details

Based on the architecture and procedures in the solution #3 of TR 23.700-3 [1], this solution consists of two types of relay discovery: one for Relay discovery among 5G ProSe UE-to-UE Relays and the other one for Relay discovery between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay.

1.	Relay discovery among 5G ProSe UE-to-UE Relays

5G ProSe UE-to-UE Relays perform a Relay discovery to form a 5G ProSe UE-to-UE Relay cloud. For the provisioning of discovery security materials and discovery message protection based on the discovery security materials associated with an RSC for multi-hop UE-to-UE Relay, the security procedures for 5G ProSe UE-to-Network Relay discovery with Model A and Model B as specified in clause 6.1.3.2.2 of TS 33.503 [5] are used with the following changes:

- 	One 5G ProSe UE-to-UE Relay plays the role of a 5G ProSe Remote UE and the other 5G ProSe UE-to-UE Relay plays the role of a 5G ProSe UE-to-Network Relay.

2.	Relay discovery between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay

The 5G ProSe End UE performs a Relay discovery to discover a 5G ProSe UE-to-UE Relay that supports a multi-hop UE-to-UE Relay. For the provisioning of discovery security materials and discovery message protection based on the discovery security materials associated with an RSC for multi-hop UE-to-UE Relay, the security procedures for 5G ProSe UE-to-Network Relay discovery with Model A and Model B as specified in clause 6.1.3.2.2 of TS 33.503 [5] are used with the following changes:

- 	A 5G ProSe End UE plays the role of a 5G ProSe Remote UE and a 5G ProSe UE-to-UE Relay plays the role of a 5G ProSe UE-to-Network Relay.

### 6.14.3	Evaluation

This solution addresses the first, third and fourth security requirements in the key issue #2 regarding the multi-hop UE-to-UE (U2U) Relay discovery.

The solution reuses the security procedure defined for Rel-17 5G ProSe UE-to-Network Relay discovery for discovery among U2U relays and discovery between a Remote UE and U2U relay.

The solution is aligned with SA2’s conclusion when PDU type IP is used for multi-hop UE-to-UE Relay service.

**** END OF CHANGES ****