3GPP TSG-SA3 Meeting #117 	S3-243396

Maastricht, Netherlands, 19 – 23 August 2024

Source:	Google

Title:	Evaluation for solution #4

Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

Approve the pCR to TR 33.701

# 2	References

[1]	3GPP TR 33.701 V0.3.0 Study on mitigations against bidding down attacks (Release 19)

# 3	Rationale

This contribution proposes a solution evaluation for solution#4 in TR 33.701.

# 4	Detailed proposal

*** Start of 1st Change ***

## 5.4	Solution #4: Solution for mitigating GERAN UTRAN bidding down attack

### 5.4.1	Introduction

This solution address KI#4.

### 5.4.2	Details

Figure 5.4.2-1: Procedure for mitigating false base station in 2G/3G decommissioning scenarios

- The core network function may send a list of 2G/3G supporting PLMNs (list-1), or a list of 2G/3G decommissioning PLMNs (list-2), or 2G/3G decommissioning information of the home network to the UE.

  - List-1: the list of 2G/3G supporting PLMNs contains the identities of the PLMNs that still deploy and maintain the radio access technologies of GERAN/GSM and/or UTRA.

    - If provided by the HPLMN, the list can contain all the PLMNs that have roaming agreement with the HPLMN and still support 2G/3G. The list may also contain the HPLMN of the UE if the HPLMN still maintains 2G/3G.

    - If provided by the serving PLMN, the list can indicate that the serving PLMN which the UE is attached to still maintains 2G/3G.

Editor's Note: Whether List-1 is sent to UE is FFS

  - List-2: the list of 2G/3G decommissioning PLMNs contains the identities of the PLMNs that have decommissioned the radio access technologies of GERAN/GSM and/or UTRA.

    - If provided by the HPLMN, the list can contain all the PLMNs that have roaming agreement with the HPLMN and have decommissioned 2G/3G. The list may also contain the home PLMN of the UE if the HPLMN has decommissioned 2G/3G.

    - If provided by the serving PLMN, the list can indicate that the serving PLMN which the UE is attached to has decommissioned 2G/3G.

  - If the home network has decommissioned 2G/3G access technologies, the 2G/3G decommissioning information may include the indication that 2G/3G has been decommissioned in the home network or the indication that UE shall not select all the GERAN/GSM/UTRA cells.

The message in step 1 may be delivered to the UE via the registration accept message or UPU procedure.

Editor's Note: it is FFS whether it is practical for an operator to maintain a list of decommissioned networks for all roaming partners

Editor's Note: it is FFS whether it is practical for a network to provide this huge list to the UEs.

Editor’s Note: Whether and how a serving PLMN should have an agreement with the PLMNs that have a roaming agreement with HPLMN is FFS.

- If the UE receives the list of 2G/3G supporting PLMNs, the UE shall only select the GSM/GERAN/UTRA cells broadcasting the PLMN IDs in list-1 (because the GSM/GERAN/UTRA cell broadcasting the PLMN ID not in list-1 is considered to be forged by a false base station).

- If the UE receives the list of 2G/3G decommissioning PLMNs, the UE shall not select the GSM/GERAN/UTRA cells broadcasting the PLMN IDs in list-2 (because the GSM/GERAN/UTRA cell broadcasting the PLMN ID in list-2 is considered to be forged by a false base station).

- If the UE receives the 2G/3G decommissioning information of the home network, the UE shall not select the GSM/GERAN/UTRA cell (because the HPLMN does not support a successful 2G/3G AKA for the UE).

NOTE 1: The information received by the UE for mitigating false base stations takes precedence over the information of PLMN/access technologies combination information configured in the UICC.

Editor’s Note: Whether and how the list can be maintained effectively such that a complete list of all roaming partners is available to the UE at the right time is FFS.

### 5.4.3	Evaluation

This solution is not work for legacy UEs.

This solution has impacts on UE.

Sending one or more of the lists impacts both the UE and the network. In particular, it impacts the UE and the AMF and the UDM function in the core network

The UE may receive list of 2G/3G supporting PLMNs (i.e. list-1).

The UE may receive the list of 2G/3G decommissioning PLMNs (i.e. list-2)

The UE may receive the 2G/3G decommissioning information of home PLMN

If home PLMN has completed the 2G/3G decommissioning, the UE shall not select the GSM/GERAN/UTRA cell (because the HPLMN does not support a successful 2G/3G AKA for the UE).

If the UE receives the list of 2G/3G supporting PLMNs, the UE shall only select the GSM/GERAN/UTRA cells broadcasting the PLMN IDs in list-1

Including information on whether the serving PLMN supports 2G/3G in List-1 is optional.

If the UE receives the list of 2G/3G decommissioning PLMNs, the UE shall not select the GSM/GERAN/UTRA cells broadcasting the PLMN IDs in list-2

This solution has impacts on registration procedure or UPU procedure.

The list of 2G/3G supporting PLMNs, list of 2G/3G decommissioning PLMNs, and the HPLMN 2G/3G decommission information may be delivered to the UE via UPU procedure or registration accept message.

In other words, the AMF includes in the Registration Accept message, one or more lists and sends it to the UE. After the UE receives the list, it can decide to establish a PDU session based on the list. Similarly, the information collected from the HPLMN may impact the UE routing parameter in the UDM of the VPLMN/SPLMN when the UE is visiting a different PLMN. The UE routing parameter information in the UDM may be sent to the UE via the AMF using the UE Parameter Update procedure. Once received, the UE will store the Updated Routing Parameter in the UICC.

If UPU is used, this solution only works in 5G.

*** End of 1st Change ***