# 3GPP TSG-SA3 Meeting #117 	S3-243535
August 19 – 23, 2024  Maastricht, NL                                                Revision of S3-242846

# Source: 	Interdigital

# Title: 	An Update for Solution #21: Remediation of unauthenticated (D)DOS in S&F

# Document for: 	Approval

Agenda Item: 	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.713

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 33.700-29 Study on Security Aspects of 5G Satellite Access

# 3	Rationale

This contribution proposes updates for Solution #21: Remediation of unauthenticated (D)DOS in S&F. Updates in Section 4 aim to resolve Editor’s Notes.

In particular, both Editor’s Notes are replaced with the explanatory text clarifying the conditions under which the additional delay and the additional round-trip are acceptable. In addition, similar considerations are added in the updated evaluation that is submitted in the companion contribution.

# 4	Detailed proposal

SA3 is asked to approve the following pCR.

*** Start of 1st Change ***

## 6.21	Solution #21: Remediation of unauthenticated (D)DOS in S&F

### 6.21.1	Introduction

This solution addresses requirement #3

The 3GPP system shall support means to mitigate the potential denial of service attack in the Store and Forward Satellite Operation.

in Key Issue #1: Security protection in Store and Forward Satellite Operation, of the present document

This solution assumes that a satellite in S&F a Service Link (SL) is available, and the Feeder Link (FL) is temporarily unavailable.

When SL and FL are not available at the same time, the satellite in S&F mode has to store NAS or AS data on board of satellite. This condition can lead to the exhaustion of storage and processing resources on board the satellite in S&F mode.

When an unauthenticated UE attempts to register, a satellite in S&F mode will need to store the registration request until the FL becomes available. This condition can be exploited by fraudulent UEs mounting a (D)DOS attack. Backoff timer and throttling are effective means to remediate (D)DOS attacks post-authentication.

This contribution proposes a solution for the remediation of unauthenticated (D)DOS in S&F.

Registration Requests can be easily spoofed by attacker(s) generating a massive number of such requests. The proposed solution is based on the idea of slowing down the rate of Registration Requests from unauthenticated UEs and it is somewhat similar to the use of CAPTCHA in WEB access. The entity that is issuing the Registration Request (i.e., a legitimate UE or an attacker) is asked to perform a kind of action that is intended to slow down the rate of the Registration Request issuance.

Such action is more expensive to perform for the entities whose goal is to issue massive amounts of such requests (i.e., an attacker) than for a legitimate UE. As a result, this solution is disproportionally affecting the attacker.

### 6.21.2	Details

The simplified call flow associated with the proposed solution is presented below.

Figure 6.21.2-1: Call flow for Solution #X: Remediation of unauthenticated (D)DOS in S&F

Steps associated with the call flow in Figure 6.21.2-1:

1. UE issues Registration Request as per Registration Procedure as per TS 23.501 Figure 4.2.2.2.2-1: Registration procedure

2. The equipment on board of satellite, (either eNB/MME or gNB/AMF) senses that there is no available Feeder Link (FL) and decides to provide a puzzle-based (D)DOS remediation

3. The equipment on board of satellite, (either eNB/MME or gNB/AMF) composes a puzzle to be solved by the UE before propagating the Registration Request further in the operator’s network

4. The equipment on board of satellite, (either eNB/MME or gNB/AMF) forwards the puzzle to the UE

5. The UE solves the puzzle and produces the evidence

6. The UE re-issues the Registration Request with the evidence of solved puzzle

7. The equipment on board of satellite, (either eNB/MME or gNB/AMF) verifies the evidence and checks for the optional freshness.

This solution (i.e., steps 2 – 4, and 7) adds the additional workload for the Network Nodes and NFs on board of satellite in the S&F mode. Such an additional workload can be an acceptable tradeoff for remediation of (D)DOS attack under the conditions where the remediation of vulnerability to both, malicious and benign (D)DOS is more important than the additional workload.

Further, this solution adds one full round-trip (i.e., steps 1-5) designed specifically to provide unauthenticated (D)DOS remediation before the AKA run. This roundtrip causes the associated delay that is the result of the proposed method of (D)DOS remediation. This additional delay represents a disadvantage of this solution.

8. The satellite detects the FL availability

9. The equipment on board of satellite, (either eNB/MME or gNB/AMF) forwards the Registration Request to the land-based CN authentication functionality (e.g., AUSF/UDM)

10. The S&F Registration Procedure continues with steps (4 or 5) as per TS 23.501 Figure 4.2.2.2.2-1: Registration procedure

*** End of 1st Change ***