SA WG3 Meeting S3#117	S3-243622

Maastricht, Netherlands 19 - 23 August 2024                    revision of S3-242873

Source:	Nokia, Nokia Shanghai Bell

Title:	Update Sol7 on Protect IMS DC based Avatar Communication

Document for:	Approval

Agenda Item:	5.2

Work Item / Release:	FS_NG_RTC_Ph2/Rel19

# 1	Decision/action requested

Approve the solution update added to TR 33.790

# 2	References

[1]	3GPP TR 23.700-77 Study on system architecture for next generation real time communication services Phase 2

[2]	3GPP TR 33.790 Study on the security support for the Next Generation Real Time Communication services phase 2

# 3	Rationale

The contribution proposes an update to existing solution#7 Protect IMS DC based Avatar to address Editor notes and add evaluation.

Chapter 6.7.2.2 and 6.7.2.3 are added to address security aspects for UE based rendering (both UE-A and UE-B based rendering) . The solution is also updated to align with SA2 conclusion on the KI, and add clarification to address other ENs and remove the corresponding ENs.

The EN related to SHAKEN framework is also removed because other NON-SHAKEN based solution will not be discussed in this solution. We will reflect the SHAKEN based option in evaluation.

# 4	Detailed proposals

Updated content in the change part is new.

# * * * * First change* * * *

## 6.7	Solution #7: Protect IMS DC based Avatar Communication

### 6.7.1	Introduction

The solution addressed KI#2 Security of IMS based Avatar Communication.

IMS avatar communication aims to provide avatar media rendered calls between the UE-A and the UE-B over IMS network. There're solutions in TR 23.700-77 proposed to use application data channel (DC) to download avatar representation (or object/metadata) for avatar media rendering. An avatar representation is stored in a data storage entity, called as Base Avatar Repository (BAR). The avatar representation is identified by an avatar id and can be fetched from the DAC using the avatar id.

This solution proposes security procedures to protect avatar id and object at rest, in transmission and in use. The solution proposes to authenticate and authorize a UE to use an avatar with signing and verifying the avatar id based on SHAKEN procedure and proposes to authenticate and authorize the XR application or MF/MRF to get avatar representation from the DAC based on CAPIF, NEF or SBI security defined in TS 33.122 and 33.501. For UE based rendering, the solution proposes to verify the authenticity of the avatar representation before using it for rendering.

### 6.7.2	Solution details

To prevent an avatar being accessed and used by unauthorized IMS caller, the solution proposes to sign and verify the avatar-id during IMS call. The solution proposes authentication and authorization based on CAPIF/NEF/SBI security when an avatar representation consumer accesses the avatar representation from the DAC. SIP security defined in 33.303 and DC integrity and confidentiality protection defined in 33.228 can be used to protect avatar id and avatar representation transmitted through IMS network.

Editor's Note: Alignment with SA2 is FFS

#### 6.7.2.1	Procedure to protect IMS DC based Avatar Communication (Network based Rendering)

Figure 6.7.2.1-1 Security procedure of IMS DC based Avatar Communication - network based rendering

1.	The UE-A initiates an IMS session and establishes audio and video session connections with the UE-B. The bootstrap data channel(s) (BDCs) are established at the same time for both the UE-A and UE-B.  The Avatar ID list is downloaded to the UE through bootstrap data channel via DCSF. The DCSF can for example retrieve the Avatar ID list from DC AS or HSS repository data as a part of subscription data in HSS

2.	The UE-A decides to request network media rendering based on its status such as power, signal, computing power, internal storage, etc. The UE-A selects the Avatar-id of the avatar from the Avatar-id list downloaded from the first step, which is intended to use for the call.

3.	As shown in the step 1 to step 26 of workflow in the AC.7.2.2 of TS 23.228, the UE-A performs the application data channel (ADC) negotiation with the XR Application Server for XR media rendering. The negotiation includes usage of the Avatar-id and the indication of network based rendering preference received from the UE-A. During this workflow, the IMS AS validates with HSS or locally based on subscription data retrieve from HSS before. If the UE is authorized to use the Avatar-id for the application based on subscription data, IMS AS signs the Avatar-id together with at least calling id, application id, then includes the signed Avatar-id in negotiation message to UE-B through the terminate IMS. The terminating IMS network verifies the signed Avatar-id. If successful, it forwards the Avatar-id to UE-B.

NOTE 1:	How to provision subscription data with Avatar-id is out of scope of this workflow.

NOTE 2:	Ms reference point can be used to sign Avatar-id together with application id. The signed Avatar-id is generated by authorized signing server based on at least calling UE IMS id (e.g. IMPU of the calling UE), Avatar-id, and application Id for the avatar call.

4.	If the negotiation result is successful in step 3, the UE-A initiates new P2A application data channels, which are used for XR data transmission between the UE-A and the network. During the P2A application data channel establishment procedure, the DCSF will instruct the MF via IMS AS how to establish the data channel and corresponding media processing specification. A UE token is included in the DC establishment messages. The UE token is generated by UE to sign UE IMS id (IMPU) and XR application server (XR AS) id with UE certificate. The fingerprint of the UE certificate can be exchanged via SIP message in step 3, which can be used to validate the UE token by IMS network. The UE token is sent to XR AS during ADC establishment or via ADC after the ADC establishment.

NOTE 3:	The UE token will be used by XR AS for authorization of accessing BAR on behalf of the UE.

5(Optional).	IMS AS initiates a media re-negotiation request with UE-A by exchanging the Avatar-id via the application DC, to connect/anchor UE-A's audio/video media stream to MF/MRF. UE-A provides to the XR Application Server via the application DC the Avatar-id of the avatar intended to use for the call.

6.	IMS AS initiates a media re-negotiation request with UE-B, to connect/anchor UE-B's audio/video media stream to MF/MRF. The Avatar-id is exchanged with UE-B to indicate about the avatar session during the signalling. UE-B has the option to reject the avatar alone or terminate the session based on Avatar-id.

NOTE 4:	Media re-negotiation in step 5 and 6 is for anchoring audio/video in MF to support network based avatar rendering.

7.	Before retrieving the avatar representation from BAR, the XR AS authenticates with NEF/CAPIF CF/NRF based on mTLS and sends token request to NEF/CAPIF CF/NRF to access avatar from the BAR. The token request includes at least signed Avatar-id, UE token, application id. NEF/CAPIF CF/NRF verifies the signed Avatar-id with IMS AS and validates the UE token based on UE certificate fingerprint exchanged in SIP message. If successfully verified Avatar-id and UE token, the NEF/CAPIF CF/NRF grants access token to the XR AS based on UE IMS Id, Avatar-Id, application id in the request and in the UE token, and policies configured locally or got from HSS. The access token includes application id, BAR instance id, Avatar-id and operations on the avatar representation associated to the Avatar-id.

NOTE 5:	Authentication and authorization mechanism defined in 33.122 or 33.501 can be reused to authenticate and authorize XR AS.

The XR Application Server retrieves the avatar representation from BAR using the Avatar-id and access token got from NEF, CAPIF CF or NRF.

NOTE 6:	The procedure assumed the BAR is inside IMS or 5GC network. It's implementation dependent if BAR is out of IMS or 5GC network.

8.	The XR Application Server retrieves the avatar representation using the Avatar-id from BAR.

9.	The BAR validates the access token and responds to the XR Application Server with the signed avatar representation.

9.1.	The XR AS verifies the signature of the avatar and expiration time of the avatar.

NOTE 7:	The certificate used to sign the avatar can be preconfigured in XR AS.

NOTE 8:	It's implementation dependent if XR AS is outside of IMS or 5GC network.

10.	The XR Application Server starts controlling the XR media rendering.

11.	The XR Application Server sends the avatar representation to MF/MRF and requests rendering of the avatar by MF/MRF.

11.1	The MF verifies the signature of the avatar and expiration time of the avatar.

NOTE 9:	The certificate used to sign the avatar can be preconfigured in the MF.

NOTE 10:	MF verifies the signature of the avatar only when XR AS is outside of IMS or 5GC network in which case the XR AS is untrusted by MF then may tamper the avatar representation.

12.	The UE-A sends information about UE-A to MF/MRF.

13.	The MF/MRF receives the information of UE-A from the UE-A and replaces the face/body with the selected avatar representation, e.g. via face detection and/or recognition mechanisms.

14.	The rendered avatar media is sent as regular video media to UE-B.

15.	The rendered avatar media is sent back to the UE-A as feedback (same content as the one sent to the UE-B in step 13), e.g. to display a thumbnail view of the avatar to the UE-A in the IMS session.

NOTE 11:	If exception happened in security steps, the IMS session may not be established or may be established without avatar media.

#### 6.7.2.2 Procedure to protect IMS DC based Avatar Communication (UE-A based Rendering)

Figure 6.7.2.1-2 Security procedure of IMS DC based Avatar Communication - UE-A based rendering

1-8. Same to steps 1-9 of Figure 6.7.2.1-1 except certificate to sign the avatar representation is sent to UE-A in step 3, UE-A based rendering indication is exchanged in step 3, and optional step 5 of Figure 6.7.2.1-1 for media re-negotiation between IMS AS and UE-A is not needed for the UE-A based rendering procedure.

9.	To ensure the avatar is not tampered especially in case the XR Application Server is not owned by the operator, the XR AS sends the signed avatar representation to the UE-A through application data channel to trigger UE-A based rendering.

10.	If the avatar representation is signed, the UE-A verifies signature of the signed avatar representation with the certificate received in step 3 from IMS AS. The UE-A locally mixes the avatar representation together with the audio/video media received from local sensors (e.g. camera) to animate the avatar (the rendered avatar audio/video media).

11.	The UE-A sends the rendered avatar audio/video media as regular video media to the UE-B.

#### 6.7.2.3 Procedure to protect IMS DC based Avatar Communication (UE-B based Rendering)

Figure 6.7.2.1-3 Security procedure of IMS DC based Avatar Communication - UE-B based rendering

1-8. Same to steps 1-9 of Figure 6.7.2.1-1 except certificate to sign the avatar representation is sent to UE-B in step 3, UE-B based rendering indication is exchanged in step 3, and optional step 5 of Figure 6.7.2.1-1 for media re-negotiation between IMS AS and UE-A is not needed for the UE-B based rendering procedure.

9.	To ensure the avatar is not tampered especially in case the XR Application Server is not owned by the operator, the XR AS sends the signed avatar representation to the UE-B through application data channel based on UE-B based rendering indication.

10.	If the avatar representation is signed, UE-B verifies signature of the signed avatar representation using the certificate received at step 3, and sends response to XR Application Server

11. After receiving the successful response from UE-B, the XR Application Server forwards the response to UE-A via IMS AS indicating about the readiness of UE-B based rendering.

12.	The UE-A sends XR information about UE-A, which is sufficient for rendering, to UE-B.

13.	The UE-B locally mixes the avatar representation together with the received information of UE-A to animate the avatar (the rendered avatar audio/video media).

# * * * * Second change* * * *

### 6.7.3	Evaluation

The solution addresses requirements of KI#2 which ensures that stored Avatar representations and Avatar-IDs are accessed only by authenticated and authorized entities, supports the integrity protection of the Avatar-ID and Avatar representation on the originating side and terminating side, as well as ensures Avatar representation will not be used for impersonating a IMS caller.

The solution covers security aspects of network based, UE-A and UE-B based avatar media rendering for data channel based avatar communication.

IMS-AS, MF, HSS, XR AS, NRF/CCF and UE need to be enhanced to protect Avatar representation and Avatar-ID at rest and in transmission, as well as ensure the Avatar can only be used by the authorized IMS user.

Editor's Note: The UE token details and the need is FFS.

## Editor's Note: Alignment with SA2 is FFS.

# * * * * End of changes * * * *