3GPP TSG-SA3 Meeting #117	S3‑243633

Maastricht, Netherlands  19 - 23 August 2024	revision of S3-243149

# Source:	Ericsson

# Title:	New solution for EEC provided information verification: UE ID token

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is proposed to add the following solution to the TR 33.749.

# 2	References

[1]	3GPP TR 33.749: "Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3".

# 3	Rationale

This document introduces a solution on verification of information provided by the EEC. The main point behind the usage of private IP address for identification of the UE is the existence of NAT after the UPF.  One of the main point in key issue #2.1 of TS 33.749 [1] is how to ensure the EEC provides the correct private IP address. This solution proposes an alternative approach for identification of UEs even if there is a NAT after the UPF.

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.749 [1].

***START OF THE CHANGE***

## 6.X	Solution #X: UE ID token

### 6.X.1	Solution overview

This solution proposes usage of UE ID token to address the security requirements key issue #2.1. With the usage of UE ID token, there will be no need to share private IP address with EEC and other entities in the EC architecture.

### 6.X.2	Solution details

High-level overview of the solution is presented in figure 6.X.2-1 and steps are explained in detail below.

Figure 6.X.2-1: UE ID token based solution

Step 1:	The EEC obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session.

Editor’s note: Further details, such as how the EEC authenticates with the UE ID server of the operator, are FFS.

Step 2:	The EEC sends the token to the EES to learn edge specific UE ID.

Step 3:	The EES invokes the NEF IP by sending the UE ID token.

Step 4:	The NEF interacts with the UE ID server to identify the UE.

Step 5:	The NEF sends the UE ID to the EES.

Step 6:	The EES sends the response to the EEC.

Editor’s note: Clarification on which architecture the solution is based on is FFS.

### 6.X.3	Solution evaluation

Editor’s note: Evaluation is FFS.

Editor’s note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

***END OF THE CHANGE***