3GPP TSG-SA3 Meeting #117	S3-243641r1

Maastricht, Netherlands  19 - 23 August 2024

merger of S3-243327, S3-242744, S3-243098 and S3-242821

# Source:	Qualcomm Incorporated

# Title:	Mobility under a TNGF

# Document for:	Approval

Agenda Item:	4.13

# 1	Decision/action requested

This contribution proposes text to capture the mobility under a TNGF.

# 2	References

None

# 3	Rationale

The below text provides details of the mobility under a TNGF using the IEEE 802.11 Fast BSS Transition (FT) procedure.

# 4	Detailed proposal

It is proposed to include the below changes in a draft CR on Non-3GPP mobility.

**** START OF CHANGES ****

### 7A.2.1	Authentication for trusted non-3GPP access

#### 7A.2.1.1	Registration with no mobility

This clause specifies how a UE is authenticated to 5G network via a trusted non-3GPP access network.

This is based on the specified procedure in TS 23.502 [8] clause 4.12a.2.2 "Registration procedure for trusted non-3GPP access". The authentication procedure is similar to the authentication procedure for Untrusted non-3GPP access defined in clause 7.2.1 with few differences, which are mentioned below:

Figure 7A.2.1-1: Registration \ Authentication and PDU Session establishment for trusted non-3GPP access

0.	The UE selects a PLMN and a TNAN for connecting to this PLMN by using the Trusted Non-3GPP Access Network selection procedure specified in TS 23.501 [2] clause 6.3.12. During this procedure, the UE discovers the PLMNs with which the TNAN supports trusted connectivity (e.g. "5G connectivity").

1.	A layer-2 connection is established between the UE and the TNAP. In case of IEEE 802.11 [80], this step corresponds to an 802.11 [80] Association. In case of PPP, this step corresponds to a PPP LCP negotiation. In other types of non-3GPP access (e.g. Ethernet), this step may not be required.

2-3.	An EAP authentication procedure is initiated. EAP messages shall be encapsulated into layer-2 packets, e.g. into IEEE 802.3/802.1x packets, into IEEE 802.11/802.1x packets, into PPP packets, etc. The UE provides a NAI that triggers the TNAP to send a AAA request to a TNGF. Between the TNAP and TNGF the EAP packets are encapsulated into AAA messages.

4-10.	An EAP-5G procedure is executed as specified in clause 7.2.1with the following modifications:

-	The EAP-5G packets shall not be encapsulated into IKEv2 packets. The UE shall also include a UE Identity Id in the AN parameter i.e., . a 5G-GUTI  or a SUCI. The value in the UE identity shall be stored at TNGF to as key identifier as described in step 13.

-	A KTNGF as specified in clause Annex A.9 (equivalent to KN3IWF) is created in the UE and in the AMF after the successful authentication. The KTNGF is transferred from the AMF to TNGF in step 10a (within the N2 Initial Context Setup Request).

-	The TNAP is a trusted entity. The TNGF shall generate the KTNAP as specified in Annex A.22 and transfers it from TNGF to TNAP in step 10b (within a AAA message).

-	After receiving the TNGF key from AMF in step 10a, the TNGF shall send to UE an EAP-Request/5G-Notification packet containing the "TNGF Contact Info", which includes the IP address of TNGF. After receiving an EAP-Response/5G-Notification packet from the UE, the TNGF shall send message 10d containing the EAP-Success packet.

11.	The common TNAP key is used by the UE and TNAP to derive security keys according to the applied non-3GPP technology and to establish a security association to protect all subsequent traffic. In case of IEEE 802.11 [80], the KTNAP is the Pairwise Master Key (PMK) and a 4-way handshake is executed (see IEEE 802.11 [80]) which establishes a security context between the WLAN AP and the UE that is used to protect unicast and multicast traffic over the air. All messages between UE and TNAP are encrypted and integrity protected from this step onwards.

NOTE 1: whether step 11 is performed out of the scope of this document. The current procedure assumes the encryption protection over Layer-2 between UE and TNAP is to be enabled.

12.	The UE receives IP configuration from the TNAN, e.g. with DHCP.

13.	The UE shall initiate an IKE_INIT exchange with the TNGF. The UE has received the IP address of TNGF during the EAP-5G signalling in step 9b, subsequently, the UE shall initiate an IKE_AUTH exchange and shall include the same UE Id (i.e. SUCI or 5G-GUTI) as in the UE Id provided in step 5. The common KTIPSe is used for mutual authentication. The key KTIPSec is derived as specified in Annex A.22.NULL encryption is negotiated as specified in RFC 2410 [81]. After step 13c, an IPsec SA is established between the UE and TNGF (i.e. a NWt connection) and it is used to transfer all subsequent NAS messages. This IPsec SA does not apply encryption but only apply integrity protection.

14.	After the NWtp connection is successfully established, the TNGF responds to AMF with an N2 Initial Context Setup Response message.

14a. The AMF may determine whether the TNGF is appropriate for the slice selected as defined in clause 4.12.2.2 of TS 23.502[8]. If it is compatible with the selected TNGF, then proceed with steps 15 to step 19. Otherwise, the AMF shall proceed with step 20 to step 22, and step 15 to step 19 are skipped.

Case a):

15.	Finally, the NAS Registration Accept message is sent by the AMF and is forwarded to UE via the established NWt connection.

16-18. The UE initiates a PDU session establishment. This is carried out exactly as specified in TS 23.502 [8] clause 4.12a.5. The TNGF may establish one or more IPSec child SA’s per PDU session.

19. User plane data for the established PDU session is transported between the UE and TNGF inside the established IPSec child SA

Case b:)

20. The AMF may trigger the UE policy update procedure and update the UE policy as defined in step 17 and step 18 in clause 4.12a.2.2 of TS 23.502[8].

21. The AMF shall send a Registration Reject message via TNGF to the UE as defined in step 19 to step21 in clause 4.12a.2.2 of TS 23.502[8]. The Registration Reject message is ciphered and integrity protected.

22. The UE shall decipher and verify the integrity of the Registration Reject message. If verification is successful, then the UE proceeds with step 21 in clause 4.12.2.2 of TS 23.502[8], and sends a Registration request message to the AMF via a new selected TNGF.

#### 7A.2.1.2	Mobility procedure in TNAN

Editor’s note: It is FFS whether to use a new key KFT as derive the  FT keys or relying on the existing key KTNAP to perform this role. The changes currently reflect the former case. To change to the latter case require the removal of Annex A.22 as no new key derivation is needed, some re-wording in clauses 7A.2.1.2 and 7A.2.3 to reflect that KTNAP rather than KFT is used.

This clause specifies the TNAP mobility procedure based on the IEEE 802.11 Fast BSS Transition (FT) procedure [80]. The mobility procedure specified in this clause is applicable to devices supporting IEEE 802.11 technologies.

When TNAN supports the FT, the TNAP in the TNAN advertises the FT capability in the Beacon and Probe Response frames by including the Mobility Domain Element (MDE). The MDE is advertised in the Beacon and Probe Response frames to indicate the Mobility Domain ID (MDID), FT capability, and the FT policy.

For trusted non-3GPP access via TNAN supporting FT, the UE and TNGF perform the following in addition to the procedures specified in clause 7A.2.1.1 of the present document. If the TNAN supports the FT,  the TNGF shall derive a FT key, KFT, from KTNGF as specified in A.22 of the present document and send the KFT to the R0 key holder (R0KH) in TNAN. If the UE supports the FT and connects to the TNAN supporting the FT, the UE shall derive a FT key, KFT, from KTNGF as specified in A.22 of the present document. The UE shall use KFT as Master PMK (MPMK) to construct the FT key hierarchy as specified in [80].

NOTE 1: The R0KH takes the received KFT as MPMK and constructs the FT key hierarchy.

When the UE switches to a new TNAP within the same mobility domain identified by the MDID, the UE performs the FT procedure as specified in IEEE 802.11 [80]. The R0KH derives a key PMK-R1 from PMK-R0 and provides the key PMK-R1 to the new TNAP in TNAN during the FT procedure.

**** NEXT CHANGES ****

### 7A.2.3	Key hierarchy for trusted non-3GPP access

The key hierarchy described in clause 6.2.1 applies, with the following changes:

The key derived for non-3GPP access is called KTNGF in the context of trusted access.

The key KTNGF received from AMF is used for three different purposes; to setup IPSec SAs between the UE and the TNGF and to create WLAN keys between the UE and the TNAP and to create the root key for IEEE 802.11 Fast BSS Transition (FT).

To separate the keys for these purposes, the key hierarchy in Figure 7A.2.3-1 shall be used. The KTIPSec key is used to setup IPSec SAs, the KTNAP key is used to setup access security and the KFT key is used as the root key for FT.

The keys KTIPSec, KTNAP and KFT are derived as described in Clause A.22.

Figure 7A.2.3-1 Key hierarchy for trusted non-3GPP access

**** NEXT CHANGES ****

# A.22	KTIPSec, KTNAP and KFT derivation function

When deriving a KTIPSec from KTNGF and when deriving a KTNAP or KFT from KTWIF or KTNGF the following parameters shall be used to form the input S to the KDF.

-	FC = 0x84

-	P0 = Usage type distinguisher

-	L0 = length of Usage type distinguisher (i.e. 0x00 0x01)

The values for the Usage type distinguisher are defined in table A.22-1. The values 0x00 and 0x04 to 0xf0 are reserved for future use, and the values 0xf1 to 0xff are reserved for private use.

The Usage type distinguisher shall be set to the value for IPSec (0x01) when deriving KTIPSec. The Usage type distinguisher shall be set to the value for TNAP (0x02) when deriving KTNAP. The Usage type distinguisher shall bet set to the value for FT (0x03) when deriving KFT.

The input key KEY shall be the 256-bit KTNGF or KTWIF.

Table A.22-1: Usage type distinguishers

| Usage type distinguisher | Value |
| --- | --- |
| IPSec | 0x01 |
| TNAP | 0x02 |
| FT | 0x03 |

**** END OF CHANGES ****