3GPP TSG-SA3 Meeting #117	S3-243668

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Ericsson

# Title:	Solution for multiple USS case using token

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

It is proposed to add the following solution to the TR 33.759.

# 2	References

[1]	3GPP TR 33.759: " Study on security enhancements of Uncrewed Aerial Systems (UAS) Phase 3".

# 3	Rationale

The document proposes a new solution for key issue #1 (security enhancements to NEF services in support of multiple USSs) in TR 33.759 [1].

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.759 [1].

***START OF THE CHANGE***

## 6.Y	Solution #Y: UUAA and authorization of target USS using token for the multiple USS case

### 6.Y.1	Introduction

This solution addresses key issue #1 (security enhancements to NEF services in support of multiple USSs) by providing enhancements to the UUAA and authorization of target USS in invoking exposure services for the UAV.

### 6.Y.2	Solution details

#### 6.Y.2.1	Identification of USS-level changeover need

For identification of changeover need, USS-A can use a planned flight path of the UAV, some data analytics to predict the location of the UAV, use the information of intersection of geographical areas served by both USSes, or can rely on notifications from the UAS NF.

Editor’s Note: FFS how to ensure AA with USS-B is in time before changeover?

#### 6.Y.2.3	Enhancement for UUAA

When the serving USS (USS-A) that has performed the UUAA procedure with the UAV identifies that USS-level changeover with a target USS (USS-B) is necessary, then

- USS-A informs the UAV, sends information about USS-B and provides a token to be used towards USS-B.

- The UAV performs authentication and authorization using the information and token.

- USS-B informs the network about the authentication and authorization result.

Editor’s Note: How to obtain "token" and whether it is in scope is FFS.

#### 6.Y.2.5	Enhancement for authorization of target USS to use exposure services for the UAV

When the serving USS (USS-A) that has performed the UUAA procedure with the UAV identifies that USS-level changeover with a target USS (USS-B) is necessary, then

- USS-A informs USS-B about the changeover and sends token also including information about the UAV.

- USS-B sends the token to the UAS NF.

- The UAS NF verifies the token and stores the information that USS-B is authorized to consume the exposure services for the UAV.

- When the UAS NF receives a request from USS-B about UAV, the UAS NF authorizes USS-B by using the stored information and checks whether the location of the UE is inside the geographical area served by USS-B.

Editor’s Note: How to obtain "token" and whether it is in scope is FFS.

Editor’s Note: Can UAS NF trust info sent by USS-A that USS-B is trustworthy is FFS. Any threat introduced to CN is FFS.

### 6.Y.3	Evaluation

TBD

***END OF THE CHANGE***