3GPP TSG-SA3 Meeting #117	S3-242867

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	MITRE-FFRDC, Defense Information Systems Agency EM

# Title:	New solution for KI#2: Dynamic Security Policy Enforcement Framework

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes a solution for Key Issue #2 in TR 33.794[1]: Security Policy Enforcement Framework

# 2	References

[1]	3GPP TR 33.794 Study on enablers for Zero Trust Security

[2]	3GPP TS 29.510 5G System; Network function repository services; Stage 3

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[4]	NIST Special Publication 800-207: "Zero Trust Architecture".

# 3	Rationale

This solution addresses TR 33.794 [1] KI#2 (Key Issue #2: Security mechanisms for policy enforcement at the 5G SBA); Specifically, it addresses Security policy enforcement Use Case #1: Access control decision enhancement (i.e., how the data from security monitoring can be considered in access decisions).

The existing authorization policies in the NF profile are defined in TS 29.510 [2] clause 6.1.6.2, such as allowedNfTypes, allowedSnpns, and allowedRuleSet. Additionally, the RuleSet defined in TS 29.510 clause 6.1.6.2.121 and Annex C provides more fine grain access control, such as (Allow/Deny) on a per NF instance and/or NF service (scope) level.

However, the existing authorization framework defined in TS 33.501 [3] clause 13.4.1 and policies in the NF profile are not dynamic in nature and face security drawbacks. The following bullets describe the drawbacks of this authorization framework:

- Policy has limited efficacy: Existing policy can only affect future communications and services. If a security incident is detected regarding an NF consumer, then the NF producer should update its NF profile with new authorization policy to mitigate the threat of NF consumer in the future. Thus, the NF producer will only be protected from NF consumers in the future that intiate a new discovery request or access token request with the NRF (carrying the updated NF profile). Meanwhile existing vulnerable connections to said consumer will remain unaffected (instead of being terminated, for example).

- OAuth Token lacks revocation: Revocation procedures for OAuth token are not defined, therefore NF service producer does not have a method to validate an access token belonging to a NF service consumer that has recently been determined to be untrusted.

- Management is challenging: The NF profile authorization policy must be changed for all the NFs accordingly and quickly to prevent further compromise. Second, if a human in the loop is required it may leave room for error or a window for compromise. Furthermore, the authorization policies in the NF profile do not have the option to set expiration times for temporary policies. When authorization policies and security in general become difficult to manage, they are more likely to be neglected or the risk is accepted.

Proposal: This solution proposes a standard dynamic security policy enforcement framework to address the aforementioned issues.

A logical Security Policy Distribution Function is described in this solution, which is responsible for receiving, managing, and distributing dynamic security policy to the NFs or SCP. The Security Policy Distribution Function performs some of the functions of the Policy Administrator in NIST SP 800-207 [4]. The Policy Administrator "… is responsible for establishing and/or shutting down the communication path between a subject and a resource (via commands to relevant PEPs). It would generate any session-specific authentication and authentication token or credential used by a client to access an enterprise resource.". This service is analogous of the role of NRF which authorizes and generates session specific authorization tokens for NF service access. Naturally, NRF may take the role of Security Policy Distribution Function.

# 4	Detailed proposal

SA3 is kindly requested to approve the below change to TR 33.794 [1]

********************** Start of Changes ****************************

## 7.Y	Solution #Y: Dynamic Security Policy Enforcement Framework

### 7.Y.1	Introduction

This solution addresses KI#2 (Key Issue #2: Security mechanisms for policy enforcement at the 5G SBA); Specifically, it addresses Security policy enforcement Use Case #1: Access control decision enhancement (i.e., how the data from security monitoring can be considered in access decisions).

This solution defines a security policy enforcement framework with two mechanisms to deploy dynamic security policy enforcement (i.e., Indirect and Direct Policy Enforcement).

Indirect Policy Enforcement: Is a means for the operator to send updated security policy to an SCP that will affect existing or new NF/NF communication that this SCP controls. (e.g., policy that could result in a termination of NF/NF indirect communication)

Direct Policy Enforcement: Is a means for the operator to send updated security policy directly to a NF service consumer or producer to apply directly to existing or new NF/NF communications (e.g., policy that results in termination of NF communication with a particular NF).

This solution introduces a dynamic authorization policy at the SCP or NF, which includes dynamic authorization policies such as those defined in the NF profiles (e.g.,  allowedRuleSet) but with enhancements that include expiration times and policy management capabilities. This additional dynamic authorization policy is used to supplement the authorization framework defined in TS 33.501 [3] clause 13.4.1 and does not rely on the NF profile update with the NRF.

This dynamic authorization policy is dynamic in nature and can be used for PLMN-wide security policy (e.g. NF instance ID XYZ declared not trusted, policy applicable to inter PLMN communication with a given PLMN, reduced scopes for NFs that are deemed risky). Such dynamic policies can be short lived, and after the expiration time they should no longer be applied. These dynamic policies allow time to resolve the current issues that triggered them or update the existing NF profile authorization policy (e.g., by updating the NF profiles). The dynamic authorization policy can be used as a containment measure during ongoing security incidents to reduce further compromise. For example, the dynamic authorization policies can be used to temporarily employ a directive of minimal functionality across all the NFs during a DoS attack on the PLMN core.

For indirect communication (i.e., NF communication and discovery via an SCP), authorization between the NF and the SCP is already done via local authorization policy in the SCP, as stated in TS 33.501 clause 13.3.6 “Authorization between the SCP and NFs is based on local authorization policy”. This solution proposes to enhance these local authorization policies with a dynamic authorization policy that is part of a policy management framework and is of variable length durations for narrowly-focused authorization policies. Similarly, this use of dynamic authorization policy is proposed as an additional layer of authorization policy for NFs.

The dynamic authorization policy is applied at:

- NF – when it authorizes a service request, including NRF when it authorizes an access token request, and

- SCP – when it authorizes indirect NF communication

### 7.Y.2	Solution details

Figure 7.Y.2-1: Security Policy Enforcement Framework

- NF and/or SCP subscribe to, or query (e.g., on a per NF/NF communication basis) the security policy updates from the Security Policy Distribution Function (e.g., NRF may take the role of Security Policy Distribution Function).

- The Operator Security Function (OSF) sends security policy updates to Security Policy Distribution Function when there are changes in the security policy of one or more NFs. An example of a dynamic security policies is provided in clause 7.Y.2.1.

NOTE 1: The triggering actions for sending and updated security policy is up to the operator.

- Indirect Policy Enforcement: Security Policy Distribution Function sends the security policy update to an SCP via SBI. The SCP updates the dynamic authorization policy accordingly and applies it to any ongoing or future SBA communication to enable Indirect Policy Enforcement.

- Direct Policy Enforcement: Security Policy Distribution Function sends the security policy update to the NF Service Producer or Consumer via SBI. NF service consumer or producer updates the dynamic authorization policy accordingly and applies it to any ongoing or future SBA communication to enable Direct Policy Enforcement.

#### 7.Y.2.1	Dynamic Authorization Policy details

The table 7.Y.3-1 below provides example policies for the dynamic authorization policies described in this solution. The Attributes can be taken from the NF profile authorization policies defined in TS 29.510 (e.g., RuleSet, snpns, plmns, nfTypes). The Policy ID is a unique identifier for the security policy and used for management of the policy. Distribution defines the recipients of the updated security policy. The Policy Action describe the required action (ALLOW/DENY) to be taken by the distribution entity and applied to incoming messages matching the attributes. The validity attribute defines the expiration time of this policy.

| Policy ID | Distribution | Attributes | Policy Action | Validity |
| --- | --- | --- | --- | --- |
| 1 | NRF | scopes: serviceX nfInstances: [NFInstanceA, NFInstanceB] | DENY | 2024-09-28T23:00:00.0000000Z |
| 2 | ALL | plmns: [plmnA, plmnB] | ALLOW | 2024-09-28T23:04:00.0000000Z |
| 3 | SMF, AMF | nfTypes: UPF | DENY | 2024-09-28T23:05:00.0000000Z |
| 4 | UDM (instance ID) | nfTypes: AMF | DENY | 2024-09-28T23:06:00.0000000Z |

Table 7.Y.2.1-1: Example Dynamic Authorization Security Policy

### 7.Y.4	Evaluation

This solution defines a dynamic security policy enforcement framework with two interfaces to deploy security policy enforcement (i.e., Indirect and Direct Policy Enforcement).

The services provided by the Security Policy Distribution Function (e.g., NRF) described in this solution must be specified. Services may include subscribe/notify and query/response services for dynamic security policy updates.

This solution brings enhanced 5GS functionality to support a supplemental dynamic authorization policy to enable dynamic security policy enforcement.

********************** End of Changes ****************************