SA WG3 Meeting S3#117	S3-242870

Maastricht, Netherlands 19 - 23 August 2024                    revision of S3-24wxyz

Source:	Nokia, Nokia Shanghai Bell

Title:	Update Sol9 on Secure IMS DC capability exposure

Document for:	Approval

Agenda Item:	5.2

Work Item / Release:	FS_NG_RTC_Ph2/Rel19

# 1	Decision/action requested

Approve the solution update added to TR 33.790

# 2	References

[1]	3GPP TR 23.700-77 Study on system architecture for next generation real time communication services Phase 2

[2]	3GPP TR 33.790 Study on the security support for the Next Generation Real Time Communication services phase 2

# 3	Rationale

The contribution proposes an update to existing solution#9 on Secure IMS DC capability exposure to address Editor notes and add evaluation. The contribution also updates the workflow to align with SA2 conclusion.

# 4	Detailed proposals

Updated content in the change part is new.

# * * * * First change* * * *

### 6.9.2	Solution details

Basically, the DC AS/AF is authenticated based on the description in clause 13 or clause 12 of TS 33.501. The AF authorization is based on clause 13 or clause 12 or local configuration at the NEF. In addition, the solution describes detail procedure to authorize an DC AS/AF to subscribe to DC event or control DC session in IMS network. The solution assumes the DC specific authorization policies are preconfigured in HSS or NEF, and the authorization decision will be made by NEF/HSS based on DC AS properties, DC related services and other conditions.

#### 6.9.2.1	Procedure of DC AS authorization for DC event subscription

Precondition:

-	Authorization policies are provisioned in NEF/HSS.

Figure 6.9.2.1-1 DC AS authorization for DC event subscription

Procedures:

1.	NEF receives DC event subscription request from DC AF/AS.

2.	After authenticating the AF/AS (TLS based mutual authentication), NEF retrieves authorization policies locally or from HSS.

3.	NEF makes authorization decision and grants permission for the request based on the policies and user consent. For subscriber specific event subscription request, the authorization decision may be made by HSS when it received event subscription request from the NEF.

4.	NEF sends DC non-subscriber specific event subscription request to IMS AS, or sends subscriber specific event subscription requests to HSS, then HSS forwards the request to the IMS AS.

5.	NEF receives response from IMS AS/HSS.

6.	NEF sends response to the DC AF/AS

7.	NEF receives event notification from IMS AS.

8.	NEF sends the notification to the DC AF/AS. The notification may be anonymized based on privacy policies or regulations.

#### 6.9.2.2	Procedure of DC AS authorization for data channel session control

DC session control includes bootstrap and application DC establishment, update and termination, as well as DC application download.

Figure 6.9.2.2-1 DC AS authorization for data channel session control

Procedures:

1.	NEF receives bootstrap data channel (BDC)/application DC (ADC) establishment/termination/update or application download request from DC AF/AS.

2.	After authenticating the DC AF/AS (TLS based mutual authentication), NEF retrieves authorization policies locally or from HSS.

3.	If the session control is targeted an ADC, according to authorization policies, NEF may further check if the DC AF/AS is matched to the DC application associated to the target ADC.

NOTE 1:	NEF may check locally or check with DCSF, or this check can be done by DCSF.

4.	If the DC AF/AS is allowed to perform the required session control on the DC based on the authorization policies, NEF grants permission for the request.

5.	NEF searches IMS AS from HSS and sends DC session control request to the IMS AS. IMS AS re-invites the IMS call to include DC offer with UEs, and reserve DC resource with MF if needed.

6.	NEF receives response from IMS AS.

7.	NEF sends response to the DC AF/AS.

# * * * * Second change* * * *

### 6.9.3	Evaluation

The solution addresses requirements of Key issue #3 to support authentication and authorization of data channel application server during the IMS capability exposure procedures, and also protect privacy of DC user during the IMS capability exposure procedures.

The solution is aligned with SA2 conclusion on KI#1 and KI#2 of TR 23.700-77.

# * * * * End of changes * * * *