SA WG3 Meeting S3#117	S3-242874

Maastricht, Netherlands 19 - 23 August 2024                    revision of S3-24wxyz

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution for KI#1 on Authorization supporting spatial localization service

Document for:	Approval

Agenda Item:	5.18

Work Item / Release:	FS_Metaverse_Sec/Rel19

# 1	Decision/action requested

Approve the solution added to TR 33.721

# 2	References

[1]	3GPP TR 23.700-21 Study on Application enablement architecture for mobile metaverse services

[2]	3GPP TR 33.721 Study on security aspects of 5G Mobile Metaverse services

[3]	3GPP TS 23.434 Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows

[4]	3GPP TS 33.434 Security aspects of Service Enabler Architecture Layer for Verticals (SEAL)

[5]	3GPP TS 33.122 Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs

# 3	Rationale

According to TR 23.700-21, SEAL architecture as defined in 3GPP TS 23.434 is reused for spatial anchor and spatial map management. As shown in the below figure, VAL Server/SEAL Client in the SEAL architecture plays role of metaverse Spatial Anchor management service consumer, while SEAL Server in the SEAL architecture plays role of metaverse Spatial Anchor management service producer.

Based on TS 33.434, when CAPIF is used as specified in TS 23.434, the security mechanism for CAPIF specified in TS 33.122 shall be followed.

The contribution proposes a solution for KI#1 on Authorization supporting spatial localization service based on CAPIF security mechanism.

# 4	Detailed proposals

All content is new.

# * * * * First change* * * *

## 6.X	Solution #X: Solution for KI#1 on Authorization supporting spatial localization service with CAPIF Core Function (CCF)

### 6.X.1	Introduction

The solution addresses KI#1 on Authorization supporting spatial localization service.

According to TR 23.700-21[2], SEAL architecture as defined in TS 23.434[x] is reused for spatial anchor and spatial map management. Based on TS 33.434[y], when CAPIF is used as specified in TS 23.434[x], the security mechanism for CAPIF specified in TS 33.122[z] shall be followed.

The solution proposes security procedures to authenticate and authorize the spatial anchor management service consumers.

### 6.X.2	Solution details

In TR 23.700-21[2], Service Enabler Architecture Layer (SEAL) service is enhanced to provide spatial anchor management service to the VAL server (VAL-S) or SEAL client (SEAL-C) in UE. VAL-C/SEAL-C may invoke service provided by SEAL server (SEAL-S) to create, read, update, delete or discovery spatial anchor. Based on TS 23.434[x], when CAPIF is used, the VAL server acts as CAPIF's API invoker and the SEAL server acts as CAPIF's API exposing function. CAPIF Core Function is reused in this solution to authorize spatial localization service consumer.

#### 6.X.2.1	Procedure of authorization for spatial localization service

Figure 6.x.2.1-1 Procedure of authorization for spatial localization service

0. Mutual authentication is performed between VAL server and CAPIF Core function (CCF), and secure session is established between the entities.

1. VAL server sends request to CCF to receive access token for authorization to create/read/update/delete (CRUD) or discovery spatial anchor(s) in SEAL server. The request specifies the desired operation, e.g. CRUD, spatial anchor instances to be operated, etc.

2. The CCF verifies the request's based on preconfigured policies and issues an access token to the VAL server if authorized. This token incorporates authorization attributes in different access levels.

For example,

- SEAL/VAL service level: e.g. permission to CRUD operations on metaverse localization services such as spatial anchor management service, spatial map management service.

- Spatial anchor instance level: permission to operate on specific spatial anchor instance(s)

- 3rd party service level: e.g. if multiple service information (from different service provider) is included in a spatial anchor, authorization to update all or specific service information associated with a spatial anchor.

3. The CCF returns the access token to the VAL server.

4. After mutual authentication, the VAL server establishes secure session with a SEAL server.

5. The VAL Server, equipped with the access token, sends CRUD spatial anchor request to the SEAL Server.

6. The SEAL Server verifies the access token, e.g. the issuer (CCF), subject (VAL Server), and scope (matching the requested operation).

7-8. If all validations and checks pass, the SEAL Server processes the CRUD operation on spatial anchors and sends a success response to the VAL Server.

NOTE: The security procedure is applicable to spatial map management with changing spatial anchor to spatial map.

### 6.X.3	Evaluation

TBD

# * * * * End of changes * * * *