3GPP TSG-SA3 Meeting #117																S3-242949

Maastricht, Netherlands  19 - 23 August 2024 					(revision of S3-yyxxxx)

# Source:	Nokia

# Title:	Discussion paper reauthentication via untrusted non 3GPP access

# Document for:	endorsement

Agenda Item:	4.1.15

# 1	Decision/action requested

It is requested to discuss the discussion paper on Fast BSS solution issue related to NSWO

# 2	References

[X]

# 3	Rationale

It was observed companies are not able to visualize the Fast BSS working defined in 802.11 specs. So in this we copying the relevant text from the 802.11 specs.

# 4	Detailed proposal

### 4.1 Initial context (IPsec interworking with N3IWF)

In TS 33501, section 7.2.1 Authentication for Untrusted non-3GPP Access defines authentication procedure via N3IWF and how IPSec is established between UE and N3IWF.

[Ref TS 33501 section 7.2.1] 14.	The IPsec SA is established between the UE and N3IWF by using the N3IWF key KN3IWF that was created in the UE using the uplink NAS COUNT associated with NAS connection identifier "0x02" as defined in Annex A.9 and was received by N3IWF from the AMF in step 12.

- Observation 1: As per the above reference, the IPsec is created using the KN3IWF received from the AMF.

[Ref RFC 7296 2.16.  Extensible Authentication Protocol Methods] For EAP methods that create a shared key as a side effect of authentication, that shared key MUST be used by both the initiator and responder to generate AUTH payloads in messages 7 and 8 using the syntax for shared secrets specified in Section 2.15. The shared key from EAP is the field from the EAP specification named MSK.  This shared key generated during an IKE exchange MUST NOT be used for any other purpose.

- Observation 2: As per the above reference, EAP based method can be used for authentication. Once EAP is used for the authentication, AUTH payload is generated by the shared secret (i.e. KN3IWF  ) and indirectly authentication is performed.

- Observation 3 As per the above reference, IPsec keys for encryption/integrity protection are generated using Diffie-Hellman shared secret established during that IPsec message exchange and IPsec is not using KN3IWF for encryption/integrity protection.

[Ref RFC 7296 2.18.  Rekeying IKE SAs Using a CREATE_CHILD_SA Exchange]  The CREATE_CHILD_SA exchange can be used to rekey an existing IKE SA (see Sections 1.3.2 and 2.8).  New initiator and responder SPIs are supplied in the SPI fields in the Proposal structures inside the Security Association (SA) payloads (not the SPI fields in the IKE header).  The TS payloads are omitted when rekeying an IKE SA. SKEYSEED for the new IKE SA is computed using SK_d from the existing IKE SA as follows:

SKEYSEED = prf(SK_d (old), g^ir (new) | Ni | Nr)

where g^ir (new) is the shared secret from the ephemeral Diffie- Hellman exchange of this CREATE_CHILD_SA exchange (represented as an octet string in big endian order padded with zeros if necessary to make it the length of the modulus) and Ni and Nr are the two nonces stripped of any headers.

- Observation 4 As per the above reference, IPsec nodes can perform rekeying on their own. For rekeying, the IPSec node does not need KN3IWF .

### 4.2 Reauthentication scenario added in Rel18

Lets try to understand the reauthentication via untrusted non 3GPP access

Step 0: UE is connected via N3IWF, and authentication and registration are performed as per TS 33.501 and TS 23.502. AMF provides KN3IWF to N3IWF.

Step 1: N3IWF uses KN3IWF for the AUTH payload to implicitly authenticate the UE (Step 14 of Ref TS 33501, Section 7.2.1) and complete the IPsec.

Step 2, 3: HN triggers reauthentication as per TS 33501, section ‘6.1.5 Home network triggered primary authentication procedure’.

Step 4: AMF and UE generate NAS keys and KN3IWF. The AMF may refresh NAS security via SMC.

### 4.3 Proposal

Now what will happen with the newly generated KN3IWF

| Option | Details | Pros/Cons |
| --- | --- | --- |
| Option 1: Without IPSec reauthentication | Optoin1a: AMF triggers N3IWF via ‘UE CONTEXT MODIFICATION REQUEST’ with KN3IWF, and N3IWF rekeys the IPsec SA/Child SA as per RFC 7296 without using the KN3IWF and simply discards the KN3IWF key. | Pros: Cons: KN3IWF key is transfer from AMF to N3IWF but it is not used |
|  | Option 1b: AMF triggers N3IWF via ‘UE CONTEXT MODIFICATION REQUEST’ without KN3IWF, and N3IWF rekeys the IPsec SA/Child SA as per RFC 7296. | Pros: Key is not transferred and not used Cons: |
|  | Option 1c: AMF does not trigger N3IWF, and N3IWF rekeys the IPsec SA/Child SA as per RFC 7296 when the N3IWF timer expires. | Pros: Less signalling. |
| Option2: With IPSec reauthentication | AMF triggers N3IWF via ‘UE CONTEXT MODIFICATION REQUEST’ with KN3IWF. The N3IWF needs to perform the reauthentication therefore, N3IWF needs create new SA/Child SA and all the traffic is diverted to new SA/Child Sas and old SA/Child SA are deleted. Reference: [RFC 7297 2.8.3. Rekeying the IKE SA versus Reauthentication] IKEv2 does not have any special support for reauthentication.Reauthentication is done by creating a new IKE SA from scratch (using IKE_SA_INIT/IKE_AUTH exchanges, without any REKEY_SA Notify payloads), creating new Child SAs within the new IKE SA (without REKEY_SA Notify payloads), and finally deleting the old IKE SA (which deletes the old Child SAs as well). | Cons: Not sure if traffic can be seamlessly transferred from old IPsec tunnel to new tunnel. Unnecessary overhead. |

# 5	Conclusions and proposals

Based on the observations made and option proposed, we would like to endorse the discussion paper with option 1b/1c. So that in the next meeting, SA3 and CT group can align the specs.