3GPP TSG-SA3 Meeting #117 	S3-242994

# Maastricht, NL, 19 - 23 August 2024

# Source:	Apple

# Title:	Update of Solution#10

# Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

Approve the pCR to TR 33.701

# 2	References

# 3	Rationale

This contribution proposes some update in solution#10 in TR 33.701.

# 4	Detailed proposal

*** Start of 1st Change ***

## 5.10	Solution #10: Solution for configured operator indication

### 5.10.1	Introduction

This solution addresses key issue #1: "Bidding down attacks from LTE/NR to decommissioned GERAN/UTRAN". The solution focuses on the scenario where all GERAN and UTRAN has been decommissioned in a given PLMN.

### 5.10.2	Details

If all GERAN and UTRAN has been decommissioned in a PLMN, the operator configures an indication on the UE. The indication is bound to the operator's PLMN ID and indicates to the UE that GERAN and UTRAN access technologies have been decommissioned. Based on this indication and the locatin information inside ME, when the UE is in the area of those PLMNs, UE can avoid the connection to  GERAN/UTRAN cells under those PLMNs.

When there is a false base station claiming to be a GERAN/UTRAN cell from other PLMN (i.e. not in the configured list of decommissioned RATs), UE can decline this cell based on the lcoation information.

How the indication is configured on the UE is left to out of band mechanisms or to other solutions for provisioning such information from the network. Observe, that any such mechanisms need to be secure so that the UE is not mislead into selecting still available access technologies.

NOTE: For example, the out of band mechanisms can be OTA, or other offline configuration

### 5.10.3	Evaluation

The solution addresses the requirement of key issue #1: "Bidding down attacks from LTE/NR to decommissioned GERAN/UTRAN".

The solution is limited to GERAN and UTRAN access technologies and is used only when all GERAN/UTRAN has been decommissioned.

The solution has only impact on the UE. Since the solution does not take stand on the provisioning mechanism, the impact on the network if any is for further study. In case the provisioning is left to out of band mechanisms, then network impact is limited. Furthermore, this would be beneficial even in roaming scenarios UE for serving PLMNs supporting similar out of band mechanisms.

Leaving the configuration to out of band mechanisms incurs an operational burden for the operator. Furthermore, for the roaming scenarios to work, the solution assumes an alignement of the so called out of band mechanisms. This, in practice, is very unlikely  relying on a standardized mechanism.

*** End of 2nd Change ***