3GPP TSG-SA3 Meeting #117	S3-243047

Maastricht, Netherlands 19 - 23 August 2024											(revision of S3-yyxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.328 | CR | 0078 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the requirement of media confidentiality protection for IMS data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_SEC |  |  |  |  |  | Date: |  |  | 2024-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In S3-234246 of SA3#112, IMS Data Channel security handing was added. However, 1. in clause 5.2, the usage requirement of IMS media integrity protection for RTP/MSRP and IMS data channel is not aligned. 2. In clause 5.3 of IMS media plane security features, only the use of IMS media integrity protection for IMS data channel was defined, while the use of IMS media confidentiality protection was missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Replacing "recommended" with "optional" to be aligned with the requirement on RTP and MSRP. 2. Adding the requirement of media confidentiality protection for IMS data channel, i.e., "The use of IMS media confidentiality protection for IMS data channel is optional." |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Misalignment of the usage requirement of RTP/MSRP and IMS data channel. 2. Missing the requirement of media confidentiality protection for IMS data channel. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN of 1st CHANGE ***

5.2	Media integrity protection

The support for IMS media integrity protection is mandatory in an IMS UE supporting IMS media plane security and mandatory in IMS core network elements (i.e., IMS Access Gateway) supporting DTLS-SRTP based, SDES based and/or TLS (MSRP) e2ae IMS media plane security.

The support for IMS media integrity protection is mandatory in an IMS UE supporting IMS media plane security and mandatory in IMS core network elements (i.e., Data Channel Media Function) supporting e2DCe IMS Data Channel media plane security.

The support for IMS media integrity protection is mandatory in an IMS UE and in IMS network elements, supporting e2e IMS Data Channel media plane security.

The use of IMS media integrity protection for RTP is optional, except that RTCP shall be integrity protected using SRTCP, in accordance with RFC 3711 [9].

The use of IMS media integrity protection for MSRP is optional.

The use of IMS media integrity protection for IMS data channel is optional.

*** END of 1st CHANGE ***

*** BEGIN of 2nd CHANGE ***

5.3	Media confidentiality protection

The support for IMS media confidentiality protection is mandatory in an IMS UE supporting media plane security and mandatory in IMS core network elements (i.e., IMS Access Gateway) supporting e2ae IMS media plane security.

The support for IMS media confidentiality protection is mandatory in an IMS UE supporting IMS data channel media and mandatory in IMS core network elements (i.e., Data Channel Media Function) supporting e2DCe IMS Data Channel media plane security.

The support for IMS media confidentiality protection is mandatory in an IMS UE supporting IMS data channel media and mandatory in IMS core network elements supporting e2e IMS Data Channel media plane security.

When IMS media plane security is used, SRTP transforms with null encryption should not be used.

When TLS/DTLS is used for IMS media plane security TLS/DTLS profile considerations discussed in annex M of this specification shall be followed.

The use of IMS media confidentiality protection for IMS data channel is optional.

*** END of 2nd CHANGE ***