3GPP TSG-SA3 Meeting #117	S3-243051

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Ericsson

# Title:	Update the security threats of Key Issue #2: Authorization mechanism of selection of VFL participants in the VFL group

# Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

Approve the following changes to TR 33.784 [2].

# 2	References

[1]	3GPP TR 33.738 V18.0.1 Study on security aspects of enablers for network automation for the 5G system phase 3

[2]	3GPP TR 33.784 V0.2.0 Study on security aspects of core network enhanced support for Artificial Intelligence Machine Learning (AIML)

# 3	Rationale

The threat for the unauthorized VFL server is missing in Key Issue #2: Authorization of selection of participant NWDAF instances in the Federated Learning group. And more over there are several solutions in TR 33.784 which have already addressed this threat.

# 4	Detailed proposal

*** BEGIN CHANGES ***

## 5.2	Key Issue #2: Authorization mechanism of selection of VFL participants in the VFL group

### 5.2.1	Key issue details

The TR 23.700-84 [3] studies the architecture enhancement to support VFL which allows the cooperation of  NWDAFs containing MTLF and AFs to train an ML model in 3GPP networks.

In Rel-18, security of ML model sharing between NWDAFs has been studied as a part of Horizontal Federated Learning. However, vertical federated learning(VFL) between NWDAF and AF has not been studied (e.g. when the NWDAFs and/or AFs are in different domains, locations, regions etc). Hence, the authorizations of VFL client and VFL server to participate in the VFL process shall consider the scenario that the VFL participants (i.e., VFL client and VFL server) may be the NWDAF and AF.

After successful authorization among AF(s) and NWDAF(s) in the same VFL group, the AF(s) and NWDAF(s) may share information among the members for VFL.

This key issue studies the authorization aspects of VFL server and VFL clients in the VFL group Considering AF/NWDAF can operate as a VFL client/VFL Server. VFL members also called VFL participants and VFL participant can be active, or passive as defined in TR 23.700-84 [3].

### 5.2.2	Security threats

There are following threats that could occur during the VFL process:

If a VFL client joins a VFL group without being authorized by the VFL server, it may lead to the following issues:

- 	The unauthorized VFL client may affect the generation of VFL group's ML model negatively.

If a VFL client is invited to join VFL by an unauthorized VFL server, it may lead to the following issues:

- 	The unauthorized VFL server may affect the generation of VFL group's ML model negatively.

After authorization, if NWDAF shares network internal information to the external AFs, the operator’s internal information may be exposed.

### 5.2.3	Potential security requirements

5GS shall support the authorization of members involved in vertical federated learning (VFL) group including NWDAFs and/or AFs.

5GS shall support means to avoid internal information exposed to external VFL members.

Editor’s Note: Whether the VFL group needs to be authorized is FFS which is dependent on SA2’s progress.

Editor’s Note: The terms can be updated based on SA2’s progress.

Editor’s Note: The details of network internal information, e.g. NWDAF’s NF instance ID, is ffs. Whether network internal information will be shared with external AFs is ffs and will depend on SA2’s progress. Whether details of the internal network information are considered sensitive is FFS.

*** END CHANGES ***