3GPP TSG-SA3 Meeting #117	S3-243151

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Ericsson

# Title:	Solution for multiple USS case

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

It is proposed to add the following solution to the TR 33.759.

# 2	References

[1]	3GPP TR 33.759: " Study on security enhancements of Uncrewed Aerial Systems (UAS) Phase 3".

# 3	Rationale

The document proposes a new solution for key issue #1 (security enhancements to NEF services in support of multiple USSs) in TR 33.759 [1].

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.759 [1].

***START OF THE CHANGE***

## 6.Y	Solution #Y: UUAA and authorization of target USS for the multiple USS case

### 6.Y.1	Introduction

This solution addresses key issue #1 (security enhancements to NEF services in support of multiple USSs) by providing enhancements to the UUAA and authorization of target USS in invoking exposure services for the UAV.

### 6.Y.2	Solution details

#### 6.Y.2.1	Identification of USS-level changeover need

For identification of changeover need, USS-A can use a planned flight path of the UAV, some data analytics to predict the location of the UAV, use the information of intersection of geographical areas served by both USSes, or can rely on notifications from the UAS NF.

#### 6.Y.2.2	Enhancement option #1 for UUAA

When the serving USS (USS-A) that has performed the UUAA procedure with the UAV identifies that USS-level changeover with a target USS (USS-B) is necessary, then

- USS-A informs USS-B about the changeover and sends information about the UAV.

- USS-A informs the UAV and sends information about USS-B e.g address of USS-B and security credentials.

- The UAV and USS-B uses the information provided by USS-A for authentication and authorization purpose.

- USS-B informs the network about the authentication and authorization result.

#### 6.Y.2.3	Enhancement option #2 for UUAA

When the serving USS (USS-A) that has performed the UUAA procedure with the UAV identifies that USS-level changeover with a target USS (USS-B) is necessary, then

- USS-A informs the UAV, sends information about USS-B and provides a token to be used towards USS-B.

- The UAV performs authentication and authorization using the information and token.

- USS-B informs the network about the authentication and authorization result.

#### 6.Y.2.4	Enhancement option #1 for authorization of target USS to use exposure services for the UAV

When the serving USS (USS-A) that has performed the UUAA procedure with the UAV identifies that USS-level changeover with a target USS (USS-B) is necessary, then

- USS-A informs USS-B about the changeover and sends information about the UAV.

- USS-A informs the UAS NF and sends information about USS-B and UAV.

- The UAS NF stores the information that USS-B is authorized to consume the exposure services for the UAV.

- When the UAS NF receives a request from USS-B about UAV, the UAS NF authorizes USS-B by using the stored information and checks whether the location of the UE is inside the geographical area served by USS-B.

#### 6.Y.2.5	Enhancement option #2 for authorization of target USS to use exposure services for the UAV

When the serving USS (USS-A) that has performed the UUAA procedure with the UAV identifies that USS-level changeover with a target USS (USS-B) is necessary, then

- USS-A informs USS-B about the changeover and sends token also including information about the UAV.

- USS-B sends the token to the UAS NF.

- The UAS NF verifies the token and stores the information that USS-B is authorized to consume the exposure services for the UAV.

- When the UAS NF receives a request from USS-B about UAV, the UAS NF authorizes USS-B by using the stored information and checks whether the location of the UE is inside the geographical area served by USS-B.

### 6.Y.3	Evaluation

This solution addresses the key issue #1. It supports multiple USSes.

***END OF THE CHANGE***