3GPP TSG-SA WG3 Meeting #117	 S3-243154

Maastricht, Netherlands, 19th – 23th Aug 2024

Source:	China Telecom, Xiaomi

Title:	New key issue about CAPIF interconnection

Spec:	3GPP TR 33.700-22

Agenda item:	5.19

Document for:	Approval

1. Introduction

This contribution adds new key issue about CAPIF interconnection in TR 33.700-22.

2. Reason for Change

This contribution provides the key issue needed to be considered in SA3.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 33.700-22.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[X]	3GPP TR 23.700-22: "Study on CAPIF Phase 3".

[Y]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

* * * Second Change * * * *

5.X	Key issue #X: CAPIF interconnection

5.X.1	Key issue details

TS 23.222 [Y] defines an architectural model for the CAPIF interconnection which allows API invokers of a CAPIF provider to utilize the service API(s) from the 3rd party CAPIF provider and other CAPIF core function within the same CAPIF provider. Figure 5.X.1-1 and 5.X.1-2 shows the architectural models defined in TS 23.222 [Y] clause 6.2.2.

Figure 5.X.1-1: High level functional architecture for CAPIF interconnection with multiple CAPIF provider domains

Figure 5.X.1-2: High level functional architecture for CAPIF interconnection within a CAPIF provider domain

In CAPIF interconnection, the API invoker can request authorization from the onboarded CAPIF core function (CCF), e.g., the CCF of CAPIF provider domain A in Figure 5.X.1-1, and access the service API(s) registered with other CCF, e.g., the CCF of CAPIF provider domain B in Figure 5.X.1-1. The API invoker doesn’t communicate with the CCF of CAPIF provider domain B directly. The CCFs coordinate via CAPIF-6/6e to authenticate and authorize service API invocation, which is studied in TR 23.700-22 [X] now.

It’s proposed to study the security mechanism for CAPIF interconnection. This key issue aims to study how the API invoker onboarded with one CCF is authenticated and authoirzed to access service API(s) registered in the other CCF and whether security enhencements are needed for CAPIF-6/6e.

5.X.2	Security threats

Without transmission security over CAPIF-6/6e, the messages transported via CAPIF-6/6e can be replayed, tampered with, or leaked.

Without the API invoker authentication mechanism in CAPIF interconnection scenarios, a malicious API invoker can impersonate another victim API invoker to access service API(s) registered in the other CCF.

Without the API invoker authorization mechanism in CAPIF interconnection scenarios, a malicious API invoker can unauthorized access service API(s) registered in the other CCF.

Without the API invoker authorization revocation mechanism in CAPIF interconnection scenarios, the CAPIF system cannot withdraw the authorization for API invoker accessing service API(s) registered in the other CCF.

5.X.3	Potential security requirements

Potential security requirements for CAPIF interconnection are as followed:

-	The CAPIF shall support mutual authentication and authorization between API invoker and AEF when AEF service APIs are exposed via CAPIF-6/6e reference point in CAPIF interconnection.

-	The API invoker in one domain shall support retrieval of the security method needed for accessing AEF in another domain in CAPIF interconnection.

-	The CAPIF shall support revocation of authorization related to the API invoker in CAPIF interconnection.

-	The transport of messages over the CAPIF-6 and CAPIF-6e reference points shall be integrity protected.

-	The transport of messages over the CAPIF-6 and CAPIF-6e reference points shall be protected from replay attacks.

-	The transport of messages over the CAPIF-6 and CAPIF-6e reference points shall be confidentiality protected.

-	The CAPIF shall support mechanisms for mutual authentication between CCFs over the CAPIF-6/6e reference point.

NOTE:	Coordination with SA6 is needed.

* * * End of Changes * * * *