3GPP TSG-SA3 Meeting #117	S3-243162

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Ericsson

# Title:	Discussion on the NF consumer PLMN ID check

# Document for:	Endorsement

Agenda Item:	4.1.2

# 1	Decision/action requested

Endorse the proposal on NF consumer PLMN ID check

# 2	References

[1]	https://www.3gpp.org/ftp/TSG_SA/WG3_Security/TSGS3_93_Spokane/Docs/S3-183789.zip

[2]	https://www.3gpp.org/ftp/tsg_sa/WG3_Security/TSGS3_94_Kochi/docs/S3-190559.zip

[3]	3GPP TS 29.510 "5G System; Network function repository services; Stage 3"

[4]	3GPP TS 33.501 "Security architecture and procedures for 5G system"

[5]	SA3 CRs: CR S3-231606

[6]	CT4 CRs: C4-191528, C4-225627, C4-203256

# 3	Rationale

## 3.1	History

The introduction of NF consumer PLMN ID in the access token in Rel-15 is due to the security requirement identified by SA3 to prevent impersonation attacks by PLMN verification. There are two related LSs exchanged between SA3 and CT4.

"It is necessary to enable the receiving SEPP to reliably verify the source of the request and response message, before forwarding the message including the PLMN information to the target NF for the subsequent authorization verification procedure." [1]

"It is proposed to enable verification of the PLMN-ID of the NF consumer by including the consumer's PLMN-ID unencrypted into the access token specified in TS 29.510, e.g. as part of existing claims." [2]

As a result, CT4 add NF consumer's PLMN ID in the Access Token Claims in Rel-15 (CR C4-191528).

In Rel-17, SA3 introduce the NF consumer PLMN ID check based on 3gpp-Sbi-Originating-Network-Id header, and clarify the requirement of SCP/SEPP to include the header (CR S3-231606). Accordingly, CT4 update the 3gpp-Sbi-Originating-Network-Id header to identify the entity inserting the header (CR C4-225627).

## 3.2	Observations

Observation 1: Starting from Rel-17, there are two solutions to fulfill the security requirement regarding NF consumer PLMN ID check.

In Rel-15, NF consumer PLMN ID check is based on the access token claim.

TS 33.501 clause 13.4.1.2.2	Service Request Process:

In addition to the steps described in the non-roaming scenario in 13.4.1.1, the NF Service Producer shall verify that the PLMN-ID (or SNPN ID) contained in the API request is equal to the one inside the access token.

The pSEPP shall check that the serving PLMN ID of subject claim in the access token matches the remote PLMN ID. If PRINS is used, this can be achieved by the pSEPP checking the PLMN ID of the serving network in the access token against the PLMN ID(s) in the N32-f context.

And starting from Rel-17, NF consumer PLMN ID check based on 3gpp-Sbi-Originating-Network-Id header is introduced.

TS 33.501 clause 5.9.3.2	 Requirements for Security Edge Protection Proxy (SEPP):

Receiving SEPP behavior for the 3gpp-Sbi-Originating-Network-Id header:

- 	The receiving SEPP shall check whether the 3gpp-Sbi-Originating-Network-Id header included in the signalling message belongs to the sending SEPP’s own PLMN or SNPN. It does this by verifying that the asserted PLMN ID in the 3gpp-Sbi-Originating-Network-Id header matches one of the sending SEPP's own PLMN ID(s) or SNPN ID(s) either in the N32-f context, the sending SEPP's certificate, or a locally configured list of PLMN IDs or SNPN-IDs that the sending SEPP represents.

TS 33.501 clause 5.9.3.5	 Requirements for Network Functions (NF):

The NF that sends a signalling message (service/subscription request or notification message) shall include its PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header.

If an NF supports multiple PLMN IDs or SNPN IDs, the sending NF shall include the PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header on behalf of which the message is sent.

The handling of the PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header at the receiving NF is up to configuration and deployment.

NOTE:	A misconfigured PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header can lead to service interruption.

Observation 2: Starting from Rel-16, NF consumer PLMN ID check based on access token claim may not work when NF consumer network support multiple PLMN IDs

In Rel-16, CT4 include PLMN ID list instead of a single PLMN ID when NF supports multiple PLMN IDs in the Access Token Request to align the consistence of NRF authorization mechanism among different NRF services. (CR C4-203256).

But it is unclear how to provide one PLMN ID of the NFc in the access token claim by the NRF when it receives an access token request with a list of NFc's PLMN IDs.

It may happen that the NFc does not have an access token with a PLMN ID that match with the one contained in the API message.

According to TS 33.501, the NFp shall check the PLMN ID of access token with the PLMN ID contained in the API request (e.g., PLMN ID of 3gpp-Sbi-Originating-Network-Id header). And when the access token verification fails, NFp shall generate error response.

13.4.1.2.2           Service Request Process

In addition to the steps described in the non-roaming scenario in 13.4.1.1, the NF Service Producer shall verify that the PLMN-ID (or SNPN ID) contained in the API request is equal to the one inside the access token.

13.4.1.1.2           Service Request Process

3. If the verification is successful, the NF Service Producer shall execute the requested service and responds back to the NF Service Consumer. Otherwise, it shall reply based on Oauth 2.0 error response defined in RFC 6749 [43].

# 4	Discussion

In Rel-15, when below requirement is written, multiple PLMN IDs scenario is not considered:

In addition to the steps described in the non-roaming scenario in 13.4.1.1, the NF Service Producer shall verify that the PLMN-ID (or SNPN ID) contained in the API request is equal to the one inside the access token.

When multiple PLMN IDs are deployed in a network, any one PLMN ID from the list can represent the network. From this point of view, a list of PLMN IDs of the source network can be used as reference value to support the PLMN verification.

Based on current specification, the PLMN verification at NFp may fail when multiple PLMN IDs are deployed in the NFc's network. To simplify the discussion, let's focus on below 5 scenarios to check the problem and possible solutions in details.

Scenario 1: Two networks both support Rel-16

Scenario 2: Two networks both support Rel-17, NFc insert the 3gpp-Sbi-Originating-Network-Id header

Scenario 3: Two networks both support Rel-17, cSEPP insert the 3gpp-Sbi-Originating-Network-Id header

Scenario 4: NFc network support Rel-16, and NFp network support Rel-17

Scenario 5: NFc network support Rel-17, and NFp network support Rel-16

# 5 	Detailed proposal

Deprecate the access token-based solution if agreeable and a CR can be planned in the next meeting.

Deprecate the access token-based solution for NF Consumer PLMN ID check. The detailed CR can be discussed at the next SA3 meeting with maintenance CRs included in the agenda.