3GPP TSG-SA3 Meeting #117	S3-243336

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	Qualcomm Incorporated

# Title:	Addressing ENs in solution #10

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes to address the Editor’s Notes in solution #10.

# 2	References

[1]	TR 33.743

# 3	Rationale

This contribution addresses the Editor’s Notes in solution #10 by providing clarifications and proposing a few updates.

First, it is proposed to remove the NOTE 1 so that the Intermediate UE-to-Network relay is connected when it receives the PC5 link establishment request from the 5G ProSe Remote UE. In this case, since the step 2 is performed before step 3, the Intermediate UE-to-Network relay is considered in coverage when step 3 is initiated.

Also, for the Intermediate UE-to-Network relay to play the role of U2N Relay during UP based PC5 link establishment procedure, it needs to access to the 5G PKMF of its HPLMN. Therefore, it is proposed to add a NOTE describing an assumption that an Intermediate UE-to-Network Relay is able to access to the 5G PKMF of its HPLMN.

With the above two updates, the Intermediate UE-to-Network Relay can play the role of UN2L relay to perform UP based PC5 link establishment procedure. Therefore, it is proposed to remove the following Editor’s Note.

Editor’s Note: How the Intermediate Relay can play the role of U2N Relay to perform UP procedure while out of coverage is FFS.

Second, the Intermediate UE-to-Network relay is first authorized to provide the relay service associated with an RSC when establishing the PC5 link security with the UE-to-Network relay. After then, the Remote UE is authorized to use the relay service associated with the same RSC when establishing a PC5 link security with the authorized (upstream) Intermediate UE-to-Network Relay. Therefore, it is proposed to remove the following Editor’s Note.

Editor’s Note: How is the Remote UE authorized for connecting via U2N Relay as per UP procedure is FFS.

Third, since Remote UE/ Intermediate UE-to-Network relay(s) are authenticated and authorized to use/ provide the Relay service when establishing the PC5 link security with its upstream Intermediate UE-to-Network relay or UE-to-Network Relay, we think additional end-to-end security between the Remote UE and the UE-to-Network relay is not needed. Thus, it is proposed to remove the following Editor’s Note.

Editor’s Note: The need for a Remote UE to establish e2e security with the U2N relay is FFS.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [1].

**** START OF CHANGES ****

## 6.10	Solution #10: Multi-hop UE-to-Network Relay communication security

### 6.10.1	Introduction

This solution addresses the first, second and third security requirements in the key issue #1 regarding the multi-hop UE-to-Network (U2N) Relay communication. This solution proposes to reuse the security procedure over User Plane for PC5 security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay as specified in clause 6.3.3.2 of TS 33.503 [5]. The proposed security procedure is based on the multi-hop UE-to-Network Relay communication procedures in the solutions (i.e., solution #1, #2, and #7) of TR 23.700-03 [1].

### 6.10.2	Solution details

The security procedure for multi-hop UE-to-Network Relay communication is shown in Figure 6.10.2-1.

Figure 6.10.2-1: Security procedure for multi-hop UE-to-Network Relay communication

0.	The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC based on the procedure specified in clause 6.3 of TS 33.503 [5]. In addition, the 5G ProSe Remote UE and Intermediate UE-to-Network Relay are provisioned with UP-PRUK and UP-PRUK ID from 5G PKMF as specified in step 1 in clause 6.3.3.2.2 of TS 33.503 [5].

1.	The 5G ProSe Remote UE performs a multi-hop UE-to-Network Relay discovery procedure with the Intermediate UE-to-Network Relay and 5G ProSe UE-to-Network Relay.

2.	If the Intermediate UE-to-Network Relay does not have an existing PC5 link with the 5G ProSe UE-to-Network Relay or an intermediate UE-to-Network relay on the path to the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay establishes a PC5 link with the 5G ProSe UE-to-Network Relay or the intermediate UE-to-Network relay based on the PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane specified in clause 6.3.3.2.2 of TS 33.503 [5].

Editor’s Note: How to trigger the PC5 Link establishment with security is FFS.

3.	The 5G ProSe Remote UE establishes a PC5 link with the Intermediate UE-to-Network Relay based on the PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane specified in clause 6.3.3.2.2 of TS 33.503[5] with the Intermediate UE-to-Network Relay taking the role of the 5G ProSe UE-to-Network Relay.

NOTE x: It is assumed that an Intermediate UE-to-Network Relay is able to access to the 5G PKMF of its HPLMN.

Editor’s Note: How the Intermediate Relay decides to connect to U2N Relay without connection request from Remote UE is FFS.

Editor’s Note: Alignment with SA2 conclusion on the procedures is FFS.

### 6.10.3	Evaluation

TBD

**** END OF CHANGES ****