3GPP TSG-SA3 Meeting #117	S3-243337

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	Qualcomm Incorporated

# Title:	Adding an evaluation to solution #10

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes to add an evaluation in solution #10.

# 2	References

[1]	TR 33.743

# 3	Rationale

This contribution proposes an evaluation text in solution #10. Based on the updates proposed in S3-243335 and S3-243336, this solution provides PC5 hop link security for multi-hop UE-to-Network Relay by simply reusing the PC5 link establishment security over user plane for single-hop UE-to-Network Relay defined in Rel-17. Also, the updated solution is aligned with the SA2 conclusion on the procedure when Model A based discovery is used. Therefore, it is proposed to add an evaluation text captures the above aspects.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [1].

**** START OF CHANGES ****

### 6.10.3	Evaluation

This solution addresses the security requirements in the key issue #1 regarding the multi-hop UE-to-Network (U2N) Relay communication by reusing the security procedure over use plane for single-hop 5G ProSe U2N Relay communication.

This solution is aligned with the SA2 conclusion when Model A based discovery is used.

**** END OF CHANGES ****