3GPP TSG-SA3 Meeting #117	S3-243412

Maastricht, Netherlands  19 - 23 August 2024

Title:	Reply LS on clarification on home network triggered re-authentication

Response to:	LS S3-243391/C4-235577 on clarification on home network triggered re-authentication from CT4

Work Item:	HN_Auth

Source:	SA3

To:	CT4

Cc:	-

Contact person:	Vlasios Tsiatsis

vlasios.tsiatsis@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

SA3 would like to thank CT4 for the questions.

From an SA3 perspective, the AMF will respond either with an acknowledgement message or a failure message.

The acknowledgement message is to tell the UDM that the AMF is currently handling or will initiate the primary authentication. The response message is used to tell the UDM that the AMF will not initiate the primary authentication based on the request.

With respect to the specific Questions asked by the incoming LS, below are the provided responses.

Question 1: Should the above user cases be considered as valid failure cases?

Answer 1:

From an SA3 perspective, case #2 shall be treated as failure case, the (source) AMF informs the UDM with a failure response. The UDM based on local policy can decide to wait for a subsequent AMF registration in UDM (e.g. after Handover is completed) before retrying to trigger authentication with another AMF in another access or with the new AMF that registers in the same access.

In any case for cases #1 and #4, the AMF receiving the re-authentication notification message sends acknowledgement response message to the UDM.

Case #3 is understood to be the same as Case#2 since from the UDM point of view the Re-Authentication Notification is always sent to the AMF currently registered. Therefore, the AMF can respond with a failure cause which is the same as the failure cause returned by case #2.

For case #5 is a failure case and the UDM can take immediate action as already specified in 33.501, 6.1.5 (e.g. try AMF of another access).

Question 2: Does SA3 see any need of differentiated handling in the UDM in each failure case? Or, what’s the expected UDM behaviour from the SA3 point of view?

Answer 2: From SA3 perspective, when UDM requests to authenticate the UE, it means a situation that can only be addressed by authenticating the UE. The UDM handles the cases of the acknowledgement and the failures according to the existing specification in 33.501, 6.1.5. In the case of failures, the UDM may handle differently the failure case resulting from case #2 and case #5 above. Therefore, the AMF should respond with two different types of failure causes for the above scenarios.

# 2	Actions

To CT4

ACTION: SA3 kindly requests CT4 to take the above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 October 2024		Hyderabad (India)

SA3#119	11 - 15 November 2024		Orlando (US)