3GPP TSG-SA3 Meeting #117	S3-243419

Maastricht, Netherlands  19 - 23 August 2024

# Source:	BSI (Federal Office for Information Security)

# Title:	Add specific UDR SCAS test cases for TS 33.530

# Document for:	Approval

Agenda Item:	4.2

# 1	Decision/action requested

This contribution proposes UDR-specific SCAS test cases for the authentication status.

# 2	References

[1]	3GPP TS 33.530 Security Assurance Specification (SCAS) for the Unified Data Repository (UDR) v0.1.0

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements"

# 3	Rationale

This contribution proposes UDR-specific test cases related to security functional requirements of the UDR to draft TS [1] chapter 4.

These two test cases come from the requirements described in 5.8b Requirements on the UDR (DraftCR S3-242662 for release 19) and the threat references as part of the Annex (DraftCR S3-24266). It can be argued that these test cases, much like the requirements and threats, are inherited from the UDM test cases. The UDM can use the UDR as a repository function for storing data. In these cases, the UDR must work correctly with respect to the requirements. During AKA, the UDR fulfills a critical role in updating the sequence number, but also in storing the authentication status of the UE.

It must be clearly stated that these test cases are security related test cases. Failure of the UDM to meet these requirements for storing the authentication status or updating the authentication data correctly has security implications. For example, failure to update the sequence number can lead to a denial of service in the event of a synchronization failure. If the authentication status is not stored correctly, attacks can bypass the increased home control.

# 4	Detailed proposal

It is proposed that SA3 approve the below changes for inclusion in the TS [1].

********** START OF 1st CHANGE **********

[4]		3GPP TS 33.501: "Security architecture and procedures for 5G system"

[5]		3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP

network product classes".

[6]	3GPP TS 29.504: “5G; 5G System; Unified Data Repository Services; Stage 3”

********** END OF CHANGE **********

********** START OF 2nd CHANGE **********

## 4.2	UDR-specific adaptations of security functional requirements and related test cases

## 4.2.1 		Authentication and key agreement procedure

## 4.2.1.1 Patch authentication data

Requirement Name: Failure to update UE security information

Requirement Reference: TS 33.501 [4], clause 5.8b Requirements on the UDR

Threat References: TR 33.926 [5], clause AA 2.2.1 Failure to store and delete authentication status

Requirement Description: The UDR updates security-related information such as the SQN for resynchronisation.

Test Case:

Test Name: TC_VERIFY_UPDATE_SQN_UDR

Purpose:

- Verify that the UDR under test updated the SQN correctly.

Procedure and execution steps:

Pre-Condition:

- The test case is applicable for the case where UDR and UDM are implemented as separate network functions (non-colocated)

- The tester shall have access to the UE credentials

- The UDM might be simulated

Execution Steps:

1) The tester retrieves the authentication data of the UE from the UDR using the Authentication Data endpoint.

2) The tester updates the SQN of the UE to a new value at the UDR using the Authentication Data endpoint with a patch command.

3) The tester retrieves the authentication data of the UE from the UDR using the Authentication Data endpoint.

Expected Result:

The authentication data from the last request contains the updated SQN.

Expected format of evidence:

Suitable evidence for the interface could include a packet trace, such as a pcap file.

### 4.2.1.2 Storing of authentication status of UE

Requirement Name: Storing of authentication status of UE by UDM.

Requirement Reference: TS 33.501 [4], clause 5.8b Requirements on the UDR

Threat References: TR 33.926 [5], clause AA 2.2.2 Failure to store and delete authentication status

Requirement Description: The UDR updates and stores the security relevant data of the UE.

Test Case:

Test Name: TC_PUT_GET_DEL_STATUS_UDR

Purpose:

-	Verify that the UDR under test stores the authentication status of UE.

Procedure and execution steps:

Pre-Condition:

- The test case is applicable for the case where UDR and UDM are implemented as separate network functions (non-colocated)

-	The tester shall have access to the UE credentials

-	The tester knows if UDR supports the feature PerUePerSnAuthStatus as defined in TS 29.504 [6] clause 6.1.8 Feature negotiation.

- The network environment, such as the UDM, can be simulated.

Execution Steps:

Test A:

1) The tester stores the authentication status data of a UE in the UDR using the Authentication Status endpoint.

2) The tester retrieves the authentication status of the UE from the UDR using the Authentication Status Endpoint.

Test B:

1) The tester stores the authentication status data of a UE in the UDR using the Authentication Status endpoint.

2) The tester retrieves the authentication status of the UE from the UDR using theAuthentication Status endpoint.

3) The tester deletes the authentication status data of the UE in the UDR using the Authentication Status endpoint.

4) The tester retrieves the authentication status of the UE from the UDR using the Authentication Status endpoint.

Test C:

1) This test case only applies if the UDR supports the PerUePerSnAuthStatus TS 29.504 6.1.8 Feature negotiation.

2) The tester stores the individual authentication status data of a UE in the UDR for a specific serving network using the Individual Authentication Status endpoint.

3) The tester retrieves the individual authentication status of the UE from the UDR for a specific serving network using the Individual Authentication Status endpoint.

Test D:

1) This test case only applies if the UDR supports the PerUePerSnAuthStatus TS 29.504 6.1.8 Feature negotiation.

2) The tester stores the individual authentication status data of a UE in the UDR for a specific serving network

3) The tester retrieves the individual authentication status of the UE from the UDR using the Individual Authentication Status endpoint.

4) The tester deletes the individual authentication status data of the UE in the UDR using the Individual Authentication Status endpoint.

5) The tester retrieves the individual authentication status of the UE from the UDR using the Individual Authentication Status endpoint.

Expected Results:

-	For test cases A and C, the UDM retrieves the same authentication status as sent.

-	For test cases B and D, the UDM retrieves no (individual) authentication status

Expected format of evidence:

Suitable evidence for the interface could include a packet trace, such as a pcap file.

********** END OF CHANGE **********