3GPP TSG-SA3 Meeting #117	draft_S3‑243503-r3

Maastricht, Netherlands  19 - 23 August 2024										revision of S3-243001

# Source:	Nokia, US National Security Agency, AT&T

# Title:	New solution for data collection related to API security risks

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Approve the solution update added to TR 33.790

# 2	References

[1]	3GPP TR 33.794, 'Study on enablers for Zero Trust Security', (Release 19)

# 3	Rationale

The contribution proposes a solution for collecting data which can be used to detect API security risks. This solution proposes to collect counters, KPIs, alarms and logs like detailed in Annex A.

# 4	Detailed proposals

*****Start of Change 1*****

## 7.X	Solution #X: Security data collection and exposure to enable detection of API security risks

### 7.X.1	Introduction

For SBA layer, API security risks can be considered highly likely, because these are the means of interacting with internal and external NFs. It is very important collect data for detecting any potential attacks using such risks which can lead to compromised NFs in the system.

This solution proposes the following:

- Define data dictionary for each security data point which can be collected for different kinds of API security risks. For e.g., “src_ip” for source IP, “dst_ip” for destination IP, etc..

- Assigning a unique ID to different kinds of API security risks. Such unique ID can have data types as per the security data dictionary. The actual IDs can be implementation dependent. However, assigning unique IDs can help structuring security data for analysis.

- Collecting security data points which include data associated with each API security risk.

- Raising alarms when some API security risk is detected. For example, when a burst of API access requests are received with a burst size greater than a configured threshold, an alarm can be raised to indicate potential DoS attack.

  - Security Alarms: Relevant threshold mentioned in below examples can be configured by the operators. Following can be examples of security alarms which can be raised for API related security risks:

  - Multiple simultaneous API access requests detected above threshold.

  - Such alarm can help indicate a possible API4:2023 Unrestricted Resource Consumption [2] which can lead to DoS attacks.

  - Detected usage of known vulnerability exploit.

  - Such alarm can help indicate a possible risk like API8:2023 Security Misconfiguration

  - Number of invalid tokens used for authentication exceeded threshold.

  - Such alarms can help detect a potential brute-force attack.

- Collecting security counters and KPIs which can help detection/prediction of an attack which is attempting to exploit an API security risk. Below table- X gives some examples of such counters and KPIs.

Operator’s security monitoring and evaluation functions can collect such security counters and KPIs by subscribing to NFs for receiving specific security data. The subscriptions can be dynamically changed during runtime according to monitoring policies. Only authorized function/service can subscribe to collect such data.

### 7.X.2	Solution details

Examples of security counters and KPIs which can be collected for monitoring and evaluating API security risks can be found in Table X.

Table X: Example Security Counters and KPIs

| Security Metric Name | Description | Attack |
| --- | --- | --- |
| NUM_API_INVOCATIONS | Total number of API invocations in the periodic collection interval. This can be useful for deriving some security KPIs and events related to number of API invocations. | DoS attack, API4:2023 - Unrestricted Resource Consumption |
| OUT_OF_SEQUENCE_API | Number of times out-of-sequence API is invoked in the collection interval | Reverse Engineering |
| UNAUTH_API_USER | Number of times an un-authorized user invoked an API | API Spoofing |
| SESSION_TOKEN_REUSE | Number of times session tokens are reused | Session Replay |
| AVG_API_LATENCY | This is measured by a NF invoking APIs towards other NFs. The average time taken for a NF to respond for certain API invocation is recorded here. Anomalies detected in this can indicate Man-in-the-middle attacks. In advanced security solutions, models can be trained for normal average API latencies and sequence of APIs. | Man-in-the-middle |

### 7.X.2	Evaluation

Main aspects of this solution include definition of data dictionary, defining unique ID, and collecting alarms, counters and KPIs for various API security risks.

This solution is aligned with use case #6 and aims at detailing the relevant data which should be exposed. This solution proposes collection of security counters and KPIs which can indicate exploitation of API security risks during runtime. Details of Table X can be used as examples of such counters and KPIs to be collected; however, implementations can vary and have additional or different data collections.

Editor's Note: Further evaluation is FFS.

*****End of Change 1*****

*****Start of Change 2*****

Annex A: Known API Security Risks

# A.1	Description

Following clauses provide examples of data which can be exposed to detect potential attacks performed on various APIs exposed by NFs in SBA layer. Here, the examples are considering the OWASP top 10 API security risks as a reference from [2]. However, other API security risks like reverse engineering, API spoofing, etc. can also be considered and relevant data can be exposed for security monitoring and evaluation.

The security data can be exposed so that any exploitation of such risks can be detected by security evaluation and monitoring systems. In this study, the aim is to identify what data can be exposed for such risks.

Brief descriptions of API security risks are as follows.

-	API1:2023 - Broken Object Level Authorization: Attackers can exploit API endpoints that are vulnerable to broken object-level authorization by manipulating the ID of an object that is sent within the request. Object IDs can be anything from sequential integers, UUIDs, or generic strings.

-	API2:2023 - Broken Authentication: The authentication mechanism is an easy target for attackers since it's exposed to everyone. Authentication endpoints and flows are assets that need to be protected.

-	API3:2023 - Broken Object Property Level Authorization: If APIs expose endpoints which return all object’s properties, especially for REST APIs, these properties can be misused to break the object property level authorization. When allowing a user to access an object using an API endpoint, it is important to validate that the user has access to the specific object properties they are trying to access.

-	API4:2023 - Unrestricted Resource Consumption: Multiple concurrent requests can be performed from a single local computer or by using cloud computing resources. Most of the automated tools available are designed to cause DoS via high loads of traffic, impacting APIs’ service rate.

-	API5:2023 - Broken Function Level Authorization: The best way to find broken function level authorization issues is to perform a deep analysis of the authorization mechanism while keeping in mind the user hierarchy, different roles or groups in the application. Exploitation requires the attacker to send legitimate API calls to an API endpoint that they should not have access to as anonymous users or regular, non-privileged users.

-	API6:2023 - Unrestricted Access to Sensitive Business Flows: When creating an API Endpoint, it is important to understand which business flow it exposes. Some business flows are more sensitive than others, in the sense that excessive access to them may harm the business. For example, in wireless telecom networks, charging (or billing) related business flows can be considered more sensitive for business. Exploitation usually involves understanding the business model backed by the API, finding sensitive business flows, and automating access to these flows, causing harm to the business.

-	API7:2023 – Server Side Request Forgery: Server-Side Request Forgery (SSRF) flaws can occur when an API is fetching a remote resource without validating the user-supplied URI. In general, basic SSRF (when the response is returned to the attacker), is easier to exploit than Blind SSRF in which the attacker has no feedback on whether or not the attack was successful.

-	API8:2023 - Security Misconfiguration: APIs and the systems supporting them typically contain complex configurations, meant to make the APIs more customizable. Security misconfigurations not only expose sensitive user data, but also system details that can lead to full server compromise.

-	API9:2023 - Improper Inventory Management: Threat agents can get unauthorized access through old API versions or endpoints left running unpatched and using weaker security requirements. Attackers can gain access to sensitive data, or even take over the server. Sometimes different API versions/deployments are connected to the same database with real data.

-	API10:2023 - Unsafe Consumption of APIs: Developers tend to trust data received from third-party APIs more than user input. This is especially true for APIs offered by well-known companies. Because of that, developers tend to adopt weaker security standards, for instance, in regard to input validation and sanitization. Successful exploitation may lead to sensitive information exposure to unauthorized actors, many kinds of injections, or denial of service.

-	Reverse engineering attacks using APIs: Attackers can attempt to call APIs in a reverse order than the good scenario. If the APIs are not designed to handle such error scenarios, it is likely that sensitive data is revealed in error responses. It is important to detect attackers attempting such attacks.

-	API Spoofing: In this kind of attacks, attackers attempt to portray themselves as a trusted user in order to pivot to additional users, allowing them free access to data and the ability to deal more damage without being readily discovered. These attacks often use data discovered through phishing or other such credential leaks in order to prevent other alarms, such as those found in reverse engineering, from going off.

-	Man-in-the-middle attacks: In this kind of attacks, attackers act as if they are some trusted link in the API chain, intercepting data either for morphing or offloading.

-	Replay attacks: In such attacks, attacker is rewinding time by replaying some data exchanged with APIs and forcing the server to divulge data as if the same interaction is occurring once more.

### A.1.1	Examples of data to be exposed

Below table 1 describes the data which can be exposed to detect the security risks and attacks described above. NOTE that these can be more details included in different implementations. A unique ID is suggested here to make the security data more structured and good for automated security analysis implementations.

Table 1: Data to be exposed to detect security risks and attacks

| API Security Risk / Attack | Data to be exposed to detect such security risks / attacks | Unique ID |
| --- | --- | --- |
| API1:2023 - Broken Object Level Authorization | Source NF ID, Destination NF ID, targeted object, authorization failure reason | API_OWASP2023_1 |
| API2:2023 - Broken Authentication | User ID, Time of last successful authentication, time when user ID was locked, captcha flag if present | API_OWASP2023_2 |
| API3:2023 - Broken Object Property Level Authorization | Source NF ID, Destination NF ID, targeted object, authorization failure reason | API_OWASP2023_3 |
| API4:2023 - Unrestricted Resource Consumption | Affected NF ID, number of instances of this NF ID, peak CPU usage, average CPU usage, peak number of instances, average number of instances | API_OWASP2023_4 |
| API5:2023 - Broken Function Level Authorization | Source NF ID, Destination NF ID, authorization failure reason | API_OWASP2023_5 |
| API6:2023 - Unrestricted Access to Sensitive Business Flows | Affected NF ID, access type, number of tokens reused, business flow criticality | API_OWASP2023_6 |
| API7:2023 - Server Side Request Forgery | 3rd party URI, data fetched from 3rd party, NF ID | API_OWASP2023_7 |
| API8:2023 - Security Misconfiguration | Unauthorized access to configuration | API_OWASP2023_8 |
| API9:2023 - Improper Inventory Management | Number of old versions exiting for each NF and version numbers | API_OWASP2023_9 |
| API10:2023 - Unsafe Consumption of APIs | 3rd party URI, data fetched from 3rd party, NF ID | API_OWASP2023_10 |
| Reverse Engineering Attacks | Out-of-order API calls detected | API_REV_ENG_ATTACK |
| API Spoofing attacks | Unauthorized user access attempted | API_SPOOFING_ATTACK |
| Man-in-the-middle attacks | Latency related data | API_MITM_ATTACK |
| Replay attacks | Token reuse, expired token usage, repeated message numbers, source NF IDs for such attempts. | API_REPLAY_ATTACK |

Below are some examples showing different kinds of data which can be exposed.

Security Logs: The logs can provide information about the kind of API security risk identified using keywords which can enable faster and automated analysis. Following are some examples of such logs which can be exposed:

For API1:2023 Broken Object Level Authorization from [2], following information can be included in a security log:

-	Log event description: “Broken Object Level Authorization”

-	Instead, a log event ID may also be used: Example: API_OWASP2023_1

-	NF ID attempting access to an object

-	Requested action on the object

-	Object ID (optional)

For API2:2023 Broken Authentication from [2], following information can be included in a security log:

-	Log event description: “Broken API authentication”

-	Instead, a log event ID may also be used: Example: API_OWASP2023_2

-	User ID

-	Time of last successful authentication from same user

-	Time when this user ID was locked

-	Captcha present flag (BOOLEAN, Optional)

*****End of Change 2*****