3GPP TSG-SA3 Meeting #117	S3-243687

Maastricht, Netherlands, 19 - 23 August 2024										(was S3-242772-r2)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | draftCR | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Case on No Web Server Header Info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The GSMA NESAS has reviewed and proposed changes to the existing test case via the document S3-234423. The proposed changes in this document are with reference to Slide#39. |  |  |  |  |  |  |  |  |
| Summary of change: |  | GSMA suggested changes: Define what is or is not to be used as an automatic assessment tool. As for other similar tests, these are repeating the requirement, rather than specifying the steps of the testing that is required for the test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test results and misinterpretation of test execution steps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial draft and presentation. |  |  |  |  |  |  |  |  |

******************** Start of 1st Change ************************************************************

#### 4.3.4.11	Web server information in HTTP headers

Requirement Name: Web server information in HTTP headers.

Requirement Reference: In accordance with industry best practice

Requirement Description: The HTTP header shall not include information on the version of the web server and the modules/add-ons used.

Threat References: TR 33.926 [4], clause 5.3.6.5, System Fingerprinting

Test Case:

Test Name: TC_NO_WEB_SERVER_HEADER_INFORMATION

Purpose:

To verify that HTTP headers do not include information on the version of the web server and the modules/add-ons used.

Procedure and execution steps

Pre-Conditions:

-	The tester has administrative privileges.

-	A tester machine is available.

-	The tester should have configured a script, or an automatic assessment tool adapted in line with the Requirement Description.

Execution Steps

1.	The tester checks that HTTP headers do not include information on the version of the web server and the modules/add-ons used.

NOTE 1:	The header information could be checked by examining a captured http packet or by observing the response to a manual request (e.g. curl –I <address:port>). Header fields to look for could be, but are not limited to, ‘Server’, ‘X-Powered-By’, ‘Via’ or custom header fields. Furthermore, unwanted web server information could be part of the response body (e.g. in HTML comments or meta tags) or a server banner.

NOTE 2: 	The settings responsible for limiting the header information could be checked from the webserver configuration file (e.g. Apache configuration has a ‘ServerTokens’ directive, which could be set to ‘Prod’; nginx configuration has a ‘server_tokens’ directive, which could be set to ‘off’).

Expected Results:

-	Evidence that HTTP headers do not include information on the version of the web server and the modules/add-ons used.

Expected format of evidence:

-	Log files and screen shots of test executions.

******************** End of 1st Change ************************************************************