3GPP TSG-SA3 Meeting #118	S3-243934

Hyderabad, India  14 - 18 October 2024

# Source:	Nokia, Charter

# Title:	Discussion paper for Fast BSS mobility domain and TNGF domain

# Document for:	information

Agenda Item:	4.13

# 1	Decision/action requested

It is requested to discuss the discussion paper on Fast BSS solution issue and way forward

# 2	References

[X]

# 3	Rationale

In the last meeting (SA3#117), no conclusion was made for below open points :

- TNGF domain is different than the Fast BSS mobility domain and

- FT key size related issue.

In this paper, we are trying to analyse both issues.

# 4	Detailed proposal

## 4.1 Domain Concepts understanding and observation

### 4.1.1 Domain in Fast BSS

Fast BSS transition works within a Mobility Domain. According to IEEE 802.11-2020 specification:

- mobility domain: A set of basic service sets (BSSs), within the same extended service set (ESS), that support fast BSS transitions between themselves and that are identified by the set’s mobility domain identifier (MDID).

A Mobility Domain is an area within an ESS (extended service set).

- extended service set (ESS): A set of one or more interconnected basic service sets (BSSs) that appears as a single BSS to the logical link control (LLC) layer at any station (STA) associated with one of those BSSs.

### 4.1.2 Domain in 3GPP TNGF

TNGF domain contains one or more TNAPs. TNGF and TNAP are both deployed by operators.

There is no clear definition of the TNGF domain; it is left to the operator implementation. E.g., the operator may deploy a single TNGF for a town or for a state or multiple TNGF per town or per state.

## 4.1.3 Related issue discussed in the last meeting

### 4.1.3.1 ESS domain is within the TNGF domain.

If UE is attached to TNAP/AP1 under ESS1, then

- FT will work when UE moves from TNAP/AP1a to TNAP/AP1b

- FT will not work when UE moves from TNAP/AP1a to TNAP/AP2b (ESS1 to ESS2)

- FT will work when UE moves from TNAP/AP1a to TNAP/AP3 (ESS1 to outside TNAP)

Observation 1: FT will not work with the TNGF domain if multiple ESS domains are configured within the TNGF.

### 4.1.3.2 TNGF domain is within the ESS domain.

If UE is attached to TNAP/AP1 under ESS, then

- FT will work when UE moves from TNAP/AP1a to TNAP/AP1b

- FT will work when UE moves from TNAP/AP1a to TNAP/AP2b (TNGF1 to TNGF2). If ESS boundary crosses the PLMN boundary, even then FT will work.

Observation 2: It is a problematic scenario because FT will work cross-TNGF. However, ESS and TNAP domain configurations are fully owned by telecom operators, so operators should ensure that a single TNGF domain is mapped to a single ESS domain to avoid this issue.

## FT key size understanding and observation.

In the last meeting, there was an argument that we need a new key for FT because the existing KTNAP is not 512 bits, and for FT we need a 512-bit key. However, all the existing WLAN related keys are 256-bits in 5GC. Please refer to the below reference:

- TNAP uses the 256-bit key KTNAP for 4-way handshake (reference TS 33501, section A.22).

- 256-bit MSK is used as PMK, and WLAN AP uses it for 4-way handshake. Ref TS 33501, annex S, step 18

    - Step 18 Upon receiving the EAP-Success message, the UE derives the MSK as specified in step 11, if it has not derived the MSK earlier. The UE uses the first 256-bit of MSK as PMK to perform 4-way handshake to establish a secure connection with the WLAN AN.

Secondly, as per  IEEE 802.11-2020 Clause 12.7.1.6.3 PMK-R0

…

When IEEE 802.1X authentication is used, then

— MPMK = L(MSK, 256, 256), i.e., the second 256 bits of the MSK (which is derived from the

IEEE 802.1X authentication)

…

R0-Key-Data = KDF-Hash-Length(XXKey, “FT-R0”, SSIDlength || SSID || MDID || R0KHlength ||

R0KH-ID || S0KH-ID)

PMK-R0 = L(R0-Key-Data, 0, Q)

…

XXKey is equal to MPMK

Observation 3: FT uses the ‘upper’ 256 bits of the MSK = PMK = MPMK for generation of the PMK-R0. i.e., the PMK delivered by the EAP method through IEEE 802.1X consists of 512 bits, i.e., the ‘upper’ 256 bits used by FT are already existing in the PMK coming from the EAP-Authentication.

Observation 4: If companies believe 512 bits are required for FT, then how can we generate 512 bits from the 256 keys of KTNGF? From a security standpoint, it is not a good option. Secondly, as per IEEE text, even if we provide 512 bits, MPMK will use only the second 256 bit.

Observation 5: KTNAP can be used for FT as well. We don’t need to define a new key for FT, and we can reuse the KTNAP for FT and non-FT use cases.

# 5	Conclusions and proposals

Based on observations made in this paper, it is proposed that

- Clarify and add the requirement in the normative text that operators need to design the single ESS domain for a given TNGF domain.

- Reuse the existing KTNAP key for the FT.

Solution details are available in the S3-243935