3GPP TSG-SA3 Meeting #118	S3-244010

Hyderabad, India, October 14 – 18, 2024

# Source:	OPPO

# Title:	Solution for AIoT Authentication Based on 5G-AKA

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

Approve the pCR to TR 33.713

2	References

[x]	TS 33.501 Security architecture and procedures for 5G System

[y]	TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[z]	TS 33.102 3G security; Security architecture

[zz]	R2-2406202 RAN2#126 Meeting Report

3	Rationale

This contribution proposes a 5G-AKA[x] based authentication solution for Key Issue #5.

According to the following scopes in TR 38.769[y], there are constrains such as power consumption etc.

The overall objective of the SI is to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

i.	~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

ii.	≤ a few hundred µW peak power consumption has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

For 5G-AKA, the generation of AV uses functions f1~f5 described in 3GPP TS 33.102 [z].

Figure 7: Generation of authentication vectors

Figure 9: User authentication function in the USIM

f1：Message authentication function used to compute MAC

f2：Message authentication function used to compute RES and XRES

f3：Key generating function used to compute CK

f4：Key generating function used to compute IK

f5：Key generating function used to compute AK in normal procedure

For R19 Ambient IoT device, it’s unlikely able to support so many calculations due to the power constrain. So, this solution proposes a lightweight authentication solution for Ambient IoT service.

4	Detailed proposal

SA3 is suggested to approve the following pCR.

*** Start of 1st Change ***

6.X			Solution #X: AIoT Authentication solution based on 5G-AKA

6.X.1			Introduction

The assumption of this solution is AIoT device and network sharing a long-term root key K[x].

MAC/XMAC is used for device authenticating network. MAC is calculated by network side with the K, XMAC is calculated by AIoT device side with the K.

Optionally, RES/XRES is used for network authenticating device. XRES is calculated by network side with the K, RES is calculated by device side with the K.

To provide the freshness of the authentication vectors, RAND is also used to calculate MAC/XMAC and optionally RES/XRES.

6.X.2			Details

MAC/XMAC is used for device authenticating network. MAC is calculated by network side with the K, XMAC is calculated by AIoT device side with the K.

Optionally, RES/XRES is used for network authenticating device. XRES is calculated by network side with the K, RES is calculated by device side with the K.

To provide the freshness of the authentication vectors, RAND is also used to calculate MAC/XMAC and optionally RES/XRES.

Based on 5G-AKA and RAN2 agreement for “inventory and command” case[zz], AIoT authentication solution has the following steps:

Figure 6.X.2-1 AIoT Authentication based on 5G-AKA

- Step A and Step B of the “inventory and command” case as in RAN2 agreement[zz]. After this steps AIoT NF has the AIoT device ID.

- AIoT NF sends authentication vector request to UDM/ARPF for AIoT device.

- UDM/ARPF calculates MAC with K (e.g. the root key of AIoT device) and RAND.

- Optionally, UDM/ARPF calculates XRES with K and RAND, if network wants to authenticate AIoT device.

- UDM/ARPF sends RAND, MAC, device ID and optionally XRES to AIoT Authentication Function.

- AIoT Authentication Function sends Authentication Request including RAND, MAC, device ID to AIoT NF.

- AIoT NF sends Authentication Request including RAND, MAC, device ID to Reader.

- Reader sends Authentication Request including RAND and MAC to AIoT device. Authentication Request is carried by Step C in the “inventory and command” case.

- AIoT device calculates XMAC with RAND and K.

- Optionally, AIoT device calculates RES with K and RAND.

- AIoT device verifies XMAC=MAC, then the network authentication is successful. If there is other command procedure, only after successful verification, the other command procedure aere proceeded.

- AIoT device sends Authentication Reponses to Reader, this message optionally including RES if network wants to authenticate AIoT device. Authentication Reponses is carried by Step D in the “inventory and command” case.

- Reader sends Authentication Reponses to AIoT NF.

- Optionally, AIoT NF sends Uplink Authentication Request including RES to AIoT Authentication Function.

- Optionally, AIoT Authentication Function verifies XRES=RES, then the AIoT device Authentication is successful.

As an example, the generation of MAC/XMAC and RES/XRES is described as below.

Figure 6.X.2-2 The generation of MAC and XRES at network side.

Figure 6.X.2-3 The generation of MAC and XRES at AIoT device side.

6.X.3			Evaluation

TBA.

*** End of 1st Change ***

2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service Requirements for ambient power-enabled IoT".

[3]	RP-244058, RAN New SID for Study on Solution for Ambient IoT in NR.

[4]	3GPP TR 23-700-13: "Study on Architecture Support of Ambient power-enabled Internet of Things".

[x]	TS 33.501 Security architecture and procedures for 5G System

[zz]	R2-2406202 RAN2#126 Meeting Report

*** End of 2nd Change ***