3GPP TSG-SA3 Meeting #118	S3-244232

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:		LMF authorization for AI/ML model retrieval from NWDAF

# Document for:	Approval

Agenda Item:		5.13

# 1	Decision/action requested

It is proposed to approve the following solution to TR 33.784 [1]

# 2				References

[1]	TR 33.784 V0.3.0 Study on security aspects of Core Network Enhanced Support for AIML

[2]	TS 33.501 V19.0.0 Security architecture and procedures for 5G system

# 3	Rationale

This contribution proposes a solution to KI#1 "Security aspects on enhancements to LCS to support AIML". The solution is based on Annex X.10 in 3GPP TS 33.501 [2].

# 4	Detailed proposal

*** Start Change ***

## 6.Y		LMF authorization for AI/ML model retrieval from NWDAF containing MTLF

### 6.Y.1	Introduction

The proposed solution addresses the security requirement of key issue #1. The security requirement states: "If AI model training entity and AI model consumer are different, 5GS shall support authentication and authorization of AIML model or training data retrieval between LMF and NWDAF." In addition, the requirement has a NOTE stating: "The procedures defined in TS 33.501 [5] Annex X.10 Security for AI/ML model storage and sharing needs to be taken into account.". The proposed solution reuses the principles in Annex X.10 with the difference that LMF is the entity retrieving the model from NWDAF containing MTLF, and that model storage in the ADRF is not applicable.

### 6.Y.2	Solution details

The solution is shown in Figure 6.Y.2-1.

Figure 6.Y.2-1: LMF authorization for AI/ML model retrieval from NWDAF

The solution includes the following steps:

0a.	NF Service producer, i.e. NWDAF containing MTLF, registers its NF profile in the NRF with ML model interoperability indicator per Analytics ID (or a new indication).

0b.	NF service consumer, i.e. LMF, registers at the NRF including its Vendor ID and potentially an Analytics ID or a new indication.

Editor's Note: When the service consumer is LMF, whether to use a specific analytics ID or a new indication to indicate to retrieve ML model for LMF based AI/ML positioning needs to be aligned with SA2.

1.	NF Service consumer, i.e. LMF, performs Nnrf_NFDiscovery_Request operation

2a.	NF Service consumer, i.e. LMF, requests an access token from the NRF using the Nnrf_AccessToken_Get request operation. The token request message contains, besides the parameters described in clause 13.4.1.1.2, the Vendor ID of the NF Service consumer and the Analytics ID/new indication.

2b.	NRF checks whether the NF service consumer is authorized to access the requested service in NWDAF containing MTLF. The NRF verifies that the NF Consumer's Vendor ID is included in the NF Service Producer's interoperability indicator for the Analytics ID/new indication. If the authorization is successful, the NRF grants the token including the Analytics ID/new indication as specified in the clause 13.4.1

3.	NF Service consumer performs Nnwdaf_MLModelProvision Request (Analytics ID/new indication, Vendor ID and token) service operation at the NWDAF containing MTLF to retrieve ML models for the Analytics ID.

4.	The NF Service Producer authenticates the NF Service Consumer and verifies the access token as specified in the clause 13.4.1.1.2 and ensures that the Analytics ID/new indication is included in the access token. If verification is successful, NWDAF containing MTLF determines the ML model to be shared for the requested Analytics ID/new indication.

5.	NWDAF containing MTLF sends Nnwdaf_MLModelProvision Response to the NF Service Consumer with Model ID, the address of the determined ML model.

### 6.Y.3	Evaluation

The proposed solution reuses the defined principles in Annex X.10 of TS 33.501 to authorize LMF to retrieve AI/ML from NWDAF. In this solution, the LMF is the NF Service consumer, and the AI/ML model is retrieved directly from the NWDAF, not from the ADRF; thus, the AI/ML model storage and sharing in ADRF is not included in the solution.

Editor's Note: This solution is dependent on SA2 outcome as whether to utilize an analytics ID or a different indication when the consumer is LMF, not an NWDAF, needs to be aligned with SA2.

*** End Change ***