3GPP TSG-SA3 Meeting #118	S3-244233

Hyderabad, India  14 - 18 October 2024

# Source:	Ericsson

# Title:	Resolve ENs for Solution#5

# Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

Approve the following changes to TR 33.784.

# 2	References

[1]	3GPP TR 33.784 Study on security aspects of core network enhanced support for Artificial Intelligence Machine Learning (AIML)

# 3	Rationale

Solution #5 contains the following Editor's Notes:

Editor's Note: The usage of Vendor ID is FFS.

Editor's Note: Further clarification on how AF is authenticated and authorized is FFS.

Editor's Note: AF specific information, e.g., vendor ID of AF and whether they can be included in access token is FFS.

For NEF-AF interface security, NEF performs authentication and authorization of the AF for NEF-AF interface protection as defined in TS 33.501 clause 12. In this solution, NEF also requests access token on behalf of AF for service invoking towards NWDAF to enable AF’s authorization in VFL procedure.

On the other hand, VFL makes it possible to not share models when performing FL. Then, authorization per model provider/vendor introduced for HFL is not applicable for VFL anymore. It is proposed Vendor ID is not used for VFL authorization.

Editor's Note: The role of NEF in VFL is FFS.

NEF’s roles in this solution, from security point of view, are described above. NEF may have further roles in VFL preparation, sample alignment etc. These roles are functional and under remit of SA2 WG. Thus these further potential roles of NEF are not described in this solution.

This contribution proposes to resolve the above Editor's Notes.

# 4	Detailed proposal

*** BEGIN CHANGES ***

## 6.5	Solution #5: Authorization of VFL participants involving NWDAF and AF for External AF acting as FL server

### 6.5.1	Introduction

This solution addresses Key Issue #2 "Authorization mechanism of selection of VFL participants in the VFL group" for the case External AF acting as FL server.

In this solution, the FL Server with VFL capability refers to the NWDAF/External AF that plays the role of the VFL Server, while the FL Client with VFL Capabilities refers to the NWDAF/External AF that plays the role of VFL Client, as defined in TR 23.700-84 [3].

The NRF is used as the authorization entity for the participation of VFL procedure.

When the External AF assumes the role of the VFL server, NEF registers to the NRF with the AF (VFL server) specific information (e.g., AF ID) and its VFL capability (VFL server) on behalf of the External AF. The NWDAF (VFL client) registers to the NRF with its VFL capability (VFL client) and the authorization information used for VFL procedure, e.g., allowed VFL server (External AF) related information (AF ID).

The NRF then authorizes the VFL service request from AF(VFL server) towards NWDAF(VFL Client) based on the VFL capability type (VFL server) and AF specific information (e.g., AF ID) provided by the AF(VFL server) during registration, and the VFL related authorization information (e.g. a list of  allowed AF IDs ) provided by the NWDAF(VFL client) during registration.

### 6.5.2	Solution details

Figure 6.5.2-1: VFL authorization when the External AF acts as a VFL Server

Step 1a. The NWDAF containing MTLF acting as VFL client registers to the NRF with its supported VFL capability (VFL client)and the authorization information used for VFL. It can be a list of AF specific information (e.g., AF ID) that are allowed for VFL.

Step 1b. The External AF sends registration request to the NEF to indicate that it wants to invoke VFL service by assuming the role of VFL server. In this request the external AF also sends information that can be used in the VFL participation decision such as supported VFL capability (VFL server), AF specific information (e.g., AF ID).

NOTE: 	External AF may register its data via OAM configuration at NEF.

Step 1c. The NEF registers to the NRF within its NF profile about the the AF information as specified in clause 6.2.2.3 of TS 23.288 and includes as part of the information the AF’s VFL related information, including supported VFL capability (VFL server), AF specific information (e.g., AF ID).

Step 2a, 2b. The External AF acting as VFL server sends a discovery request to NRF via NEF and receives the available NWDAFs containing MTLF acting as VFL client(s) as a response. The NEF enables this discovery procedure after authenticating and authorizing the AF.

NOTE: 	The details of how the AF selects candidate NWDAF(s) as VFL client(s) and whether the NF instance ID of NWDAF containing MTLF is sent to external AF is not described in this solution.

Step 3a. The External AF acting as VFL server sends the VFL service request (e.g., VFL preparation, sample alignment, feature alignment, VFL training, VFL inference) towards the NEF.

Step 3b. The NEF authorizes the External AF can trigger VFL service request towards NEF. The authentication and authorization between the NEF and the External AF acting as VFL server can be performed as specified in TS 33.501 [5] clause 12. If External AF is authorized, the NEF sends an access token request to the NRF to request the access token for the External AF acting as VFL server to perform VFL service request towards the VFL clients. The token request may contain the Analytics ID for the requested VFL process and also AF specific information (e.g., AF ID).

Step 4. NRF checks whether the NF Service consumer (NEF) is authorized to access the requested service in VFL client (NWDAF). In case of the NF Service Consumer (NEF) request VFL service for the External AF acting as VFL server, the NRF also verifies that the AF specific information (e.g., AF ID) is included in the authorization information used for VFL provided by the VFL client in Step 1a. If the authorization succeeds, NRF generates the access token(s) as specified in TS 33.501 [5] clause 13.4.1. The access token claims may include the Analytics ID for the request Federated Learning process. The access tokens are generated for the NEF (e.g., NEF as subject). The access token claims may include the AF specific information (e.g. AF ID) e.g. AF acting VFL server is as service consumer .

NOTE: 	Fine-grained authorization can be done locally at the NWDAFs containing MTLF acting as VFL client(s). Also, fine-grained authorization can be done locally at the NEF.

NOTE: 	Further role of NEF (e.g. in VFL preparation, sample alignment procedure) is not described in this solution.

Step 5. The NRF sends the access token to the NEF. Step 6. The NEF sends the VFL service request (e.g., VFL preparation, sample alignment, feature alignment, VFL training, VFL inference) towards the VFL clients, with the obtained token.

Step 7, 8a, 8b. The NWDAF containing MTLF acting as VFL client verifies the received access token as specified in TS 33.501 [5] clause 13.4.1. The NWDAF containing MTLF acting as VFL client may also check the AF specific information (e.g., AF ID) in the token for fine-grained authorization. In case of successful access token verification, the NWDAF containing MTLF acting as VFL client sends a success VFL service response to the External AF acting as VFL server, via the NEF.

Step 9. The Vertical Federated Learning procedure is performed between VFL server (External AF) and VFL client (NWDAF) via the NEF.

### 6.5.3	Evaluation

The solution addresses key issue #2 "Authorization mechanism of selection of VFL participants in the VFL group". It proposes to reuse NRF based authorization that NRF authorizes the VFL service request for the VFL participants based on the registered NF profile information for involving NWDAF and/or External AF.

For the case AF acts as VFL server, AF specific info is registered in NEF as part of NF profile of NEF.

When AF (VFL server) is to trigger VFL service towards NWDAF(VFL client) via NEF. The NEF authorizes the External AF as specified in TS 33.501 [5] clause 12. If External AF is authorized, the NEF request the access token on behave of the External AF(VFL server). The NRF performs authorization check and provides token that follows the same principle of HFL as specified in Annex X.9 TS 33.501. I.e. the authorization is based upon the FL capability type and AF specific information (e.g., AF ID) provided by the AF(VFL server) during registration, and the VFL related authorization information provided by the NWDAF(VFL client) during registration (e.g. a list of  allowed AF IDs ).

The access token is generated for the NEF (e.g., NEF as subject) and can also include AF specific information. So that when NWDAF(VFL client) receives the VFL service request forwarded by NEF, it can grant VFL service request from AF (VFL server) via NEF based on the token provided by NEF.

*** END CHANGES ***