3GPP TSG-SA3 Meeting #118	S3‑244382

Hyderabad, India  14 - 18 October 2024												revision of S3-244074

# Source:	Nokia

# Title:	Overall Summary of solutions

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve the overall summary of solutions added to TR 33.700-29

# 2	References

[1]	3GPP TR 33.700-29 Study on Security Aspects of 5G Satellite Access in the 5G Architecture phase 3

[2]          3GPP TR 23.700-29 Study on integration of satellite components in the 5G architecture

# 3	Rationale

The contribution proposes an overall summary of all solutions to enable conclusions of the study.

# 4	Detailed proposals

# * * * * First change* * * *

# 7	Conclusions

## 7.1	Overall Summary of Solutions

| Legend |  |
| --- | --- |
| X | Key Issue #1 Split MME S&F |
| Y | Key Issue #1 Full CN onboard S&F |
| XX | Key Issue #2 Split MME S&F |

| Solution number | No / Minimal Security Changes | Security Architecture Change | Procedural Adaptations | Novel solutions | DDoS Solutions | 5G Only solutions | Summary of key concept | Group of solutions to derive common principles for conclusions | Does it need LS? (Yes/No) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | X |  | X |  |  | Inverse AKA | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 2 | Y |  | Y |  |  |  | IOPS Security for S&F | Solutions 2, 3, 5, 12, 13 |  |
| 3 | Y |  | Y |  |  |  | IOPS Security for S&F | Solutions 2, 3, 5, 12, 13 |  |
| 4 |  |  |  | X |  | X | Bundling the authentication round trip with the NAS SMC |  |  |
| 5 | Y |  |  |  |  | Y | UDM onboard satellite | Solutions 2, 3, 5, 12, 13 |  |
| 6 | X |  | X |  |  |  | NAS Timer updates | Solutions 6, 17 |  |
| 7 |  | X |  | X |  | X | Inverse AKA for subsequent authentication | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 8 |  |  | X | X | X |  | Addresses DDoS attack on UE by False Satellite. eNB and UE generate and verify digital signatures for this. Requires a set of credentials for ECCSI is provisioned to the UE and Satellite. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 9 |  |  | X |  | X |  | Temporarily storing the UE's IMSI and issuing an Attach Reject message with a Cause value and a Timer. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 10 |  |  | X | X |  |  | UE subscription information is stored in a UE authentication token. UE provides this when it connects to satellite. Token protection keys need to be pre-configured in the satellites. | Solutions 10, 11 | Yes ? |
| 11 |  |  | X | X |  |  | Similar to solution #10. After UE successfully accesses the satellite for the first time, it can perform authentication and data transmission processes in any subsequent connection. How can the UE access satellite in S&F mode for the first time is unclear. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 | Yes ? |
| 12 | Y |  | Y |  |  |  | IOPS based solution. The HSS-NT needs to fetch the unstored user security credentials from the ground HSS when feeder link is available. | Solutions 2, 3, 5, 12, 13 |  |
| 13 | Y |  | Y |  |  |  | With HSS onboard, re-use legacy security procedures. Disadvantage of Subscriber credentials are provisioned in several satellites HSSs. Also, the key used to generate authentication vectors should be stored on the satellite, which may not be the resources of the home operator. | Solutions 2, 3, 5, 12, 13 |  |
| 14 |  | X |  | X |  |  | S&F Operation function and OTP based security solution |  |  |
| 15 | X |  |  | X | X |  | Protection against unauthorized UEs launching DDoS attacks. MME-T checks UE authorization based on subscription information from HSS before requesting authentication vectors. This solution does not address DoS attack when only service link is available and NAS/AS security context is not established. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 16 | X |  |  | X | X |  | UE authorization during S&F MO transmission. Authorization information included in UE context. How can the UE access satellite in S&F mode for the first time (i.e., unprovisioned with authorization credentials) is unclear.. This solution only addresses DoS attack after NAS/AS security context is established. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 17 | X |  | X |  |  |  | MME leverages pre-provisioned orbit information to determine the back-off timer and provide the back-off timer to the UE. Only compliant UEs will heed the backoff timer. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 18 | X |  | X |  |  |  | Re-use legacy UP Integrity procedures and discard fake data from potential attackers. |  |  |
| 19 | XX |  | XX |  |  |  | Temporary UE IDs used to generate interim GUTI |  |  |
| 20 | XX |  | XX |  |  |  | Alt1: Short Validity Period for interim GUTI Alt2: Not to use interim GUTI | Solutions 20, 30 | Yes |
| 21 |  |  |  | X | X |  | Backoff timer and throttling are effective means to remediate (D)DOS attacks post-authentication. However, the backooff timer is applicable to compliant UEs only and throttling can work only for authenticated UEs. Solution proposes puzzle solving to slow down the rate of Attach Requests from un-authenticated UEs. This solution does not require pre-provisioned specific satellite credentials into UEs. This solution does not address DoS attack on UE by false information in Attach Reject message. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 22 | X |  | X |  |  |  | Based on satellite constellation to allow UEs to connect via one of the satellites from the group. Relies on ISL connections, which is assume to be excluded in Rel-19 |  | Yes |
| 23 |  |  | X | X |  |  | Protect the RRC connection procedure via using NAS parameters from core network to ensure RAN is talking to genuine UE. |  |  |
| 24 | X |  | X |  |  | X | Pre-fetch authentication information into next satellite that will fly over UE's location. pre-fetch authentication information into next satellite that will fly over UE's location. |  |  |
| 25 |  |  | X | X | X |  | before the AKA procedure between the UE and the network is completed, the satellite and the UE check the authenticity of each other using the symmetric cryptosystem, which prevents the threats of DoS attack and false satellite attack. This solution requires satellite-specific pre-provisioned credentials. It is unclear how UEs without such credentials can attach. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 26 | XX |  | XX |  |  |  | UE pre-configured with private key and certificate for partial accept message before authentication |  |  |
| 27 | XX |  | XX |  | X |  | Preconfigured group keys to secure communication related to attach request. Once the group key from one UE is leaked, all UEs that have the key are affected. |  |  |
| 28 |  | X |  | X |  |  | The purpose of this solution is FFS. |  |  |
| 29 | Y |  | Y |  |  |  | Reuses the existing security procedures for full CN onboard. |  |  |
| 30 | XX |  | XX |  |  |  | Dynamic interim GUTI with short validity period. |  |  |

Editor’s Note: Capturing further solution specific security analysis in overall summary is FFS.

# * * * * End of changes * * * *