3GPP TSG-SA3 Meeting #118	S3-243887

Hyderabad, India,  14th - 18rd October 2024

Title:	Reply LS on AIML Data Collection

Response to:         	RP-242389 LS on AIML data collection

Release:	Rel-19

Work Item:	NR_AIML_Air

Source:	vivo (to be SA3)

To:	RAN, SA, SA2, SA5

Cc:	RAN1, RAN2, RAN3

Contact person:	Li Hu

huli@vivo.com

Send any reply LS to:   3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

SA3 would like to thank RAN for their LS RP-242389 regarding requesting inputs on solutions 1b, 2, and 3.

Following the way forward and requirements outlined in RAN#105, it is evident that requirements 1 and 2 are closely associated with the security aspect. Therefore, SA3 proceeds with the following assessments:

Table 1. Analysis of requirement 1 and 2 for option 1b, 2 and 3

|  | Option 1b | Option 2 | Option 3 |
| --- | --- | --- | --- |
| Requirement 1: data integrity and confidentiality | Out of 3GPP and rely on application layer security | CP tunnel: UE->AMF: reuse NAS security AMF->consumer: reuse SBA security UP tunnel: The same with 1b | CP tunnel: UE->gNB: reuse RRC security gNB->OAM->Server: out of SA3 UP tunnel: UE->gNB: reuse UP security gNB->OAM->Server: out of SA3 |
| Requirement 2: privacy, anonymity and user consent | Associated with MNO controllability and MNO visibility (whether MNO is data controller, and whether user consents reported data) - The MNO will handle user consent, and requirements in Annex V of TS 33.501 is applied if DCAF(Data collection server) is within MNO domain. - Out of SA3 if DCAF is within 3rd party domain. | As MNO has controllability and visibility (MNO is data controller and user consents reported data), the requirements in Annex V of TS 33.501 is applied. The CN NFs (e.g. AMF, DCNF) can be user consent enforcement point. | As MNO has controllability and visibility (MNO is data controller and user consents reported data), the requirements in Annex V of TS 33.501 is applied. The gNB or OAM can be user consent enforcement point similar like SON/MDT procedure. |

In general, SA3 does not identify any major security issues for the requirement 1 and 2 of options 1b, 2, and 3. Furthermore, many of the security solutions and requirements for data protection and privacy can be reused. SA3 emphasizes that having MNO controllability and MNO visibility of UE data are essential for user consent as the data controller has responsibility to protect user privacy and user can only consent the data reporting if only the data content  is fully visible to the user.

Actions

To RAN, SA, SA2, SA5:

ACTION: 	SA3 kindly asks RAN, SA, SA2, SA5 to take the above SA3 feedback into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#119			19 - 23 November 2024				Orlando (US)

SA3#120			14 - 18 February 2025				Athens (Greece)