3GPP TSG-SA3 Meeting #118	S3-244223

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	Conclusion for KI#1

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Approve the pCR to TR 33.759 [1].

# 2	References

[1]	3GPP TR 33.759: "Study on security enhancements of Uncrewed Aerial Systems (UAS) Phase 3"

# 3	Rationale

This contribution proposes conclusion for the KI#1 in the UAS3 TR 33.759.

# 4	Detailed proposal

*****Start of changes*****

# 7	Conclusions

For Key issue #1 on the support for the multiple USS and changeover from one USS to another, the following principles apply:

- UAS NF receives and maintains info about authorized target USS received from the serving USS during or after a UUAA procedure.

- UAS NF performs a changeover of USS with the authorized target USS during a re-authentication/authorization procedure. The UAS NF checks that the target USS matches one of the authorized USSes in the UUAA context during the procedure. The UAS NF marks the target USS as new serving USS in the UUAA context after successful re-authentication/authorization procedure. Any parameters provided by the target USS needed in the procedure for changeover verification (e.g. a token as described in clause 8.1.2 of TR 23.700-59 [2]) can be determined during the normative phase.

- UAS NF verifies that a USS requesting information of a UAV corresponds to an authorized USS in the UUAA context.

- The serving USS informs the UAV and the target USS about the changeover and also provides credentials to be used in the UUAA procedure between the UAV and the target USS.

- The serving USS informs the UAS NF that the target USS is entitled to access the information of the UAV.

*****End of changes*****