3GPP TSG-SA3 Meeting #119e 	S3-250057

# emeeting, 13 - 17 January 2025

# Source:	Apple

# Title:	Update solution#31

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

Approve the pCR to TR 33.713

- References

- TR 33.713 v0.5.0

3	Rationale

This contribution proposes update to solution#31.

4	Detailed proposal

*** Start of 1st Change ***

6.31	Solution #31: Ambient IoT ID privacy

6.31.1 	Introduction

This solution addresses KI#3 and proposes a solution to protect the privacy of the identifier of an Ambient IoT (AIoT) device. The solution focuses on the functionality in the AIoT device:

- verifying a request to share the identity of the AIoT device,

- protecting the identity of the AIoT device, when sharing it.

The procedure fits the baseline RAN procedure agreed in RAN2#125bis:

Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond;

Step B: Triggered device(s) performs the random access-like procedure, if needed;

Step C: The device may perform the data communication with the reader as needed.

6.31.2 	Solution details

Figure 6.31.2-1:  Ambient IoT ID privacy

In step 0, AIoT device(s) are configured with identity(ies) and device specific parameters, such as security credentials including device- and group-specific credentials.

In step 1, the AIoT Management Function (AIoT MF) sends a message to the reader to trigger an inventory procedure including an inventory request message and a configuration associated with the inventory procedure. The configuration includes timers (e.g., to broadcast the request and to collect responses from AIoT device(s)), response aggregation requirements and forwarding criteria, etc.

Note 1: The AIoT Management Function (AIoT MF) depends on the entity that owns or manages the resource (i.e., AIoT device). If a device is owned or managed by the network, the disabling is triggered by a CN function, otherwise, it is triggered by a 3rd party Management Function managing the device.

In step 2, the reader UE/gNB sends the inventory request message, as received in step 1, to AIoT device(s). The inventory request includes a device (or group) identifier and a device-specific (or group-specific) challenge. Based on the configuration received, the reader UE/gNB may start a timer T1 to collect responses.

In step 3, The AIoT device processes the received request and checks whether the message:

3.1 is addressed to the AIoT device by matching its identifier (i.e., device or group identifier) against the received identifier, and

3.2 is sent by a trusted party by verifying the challenge value using its security credentials and identifiers.

If the checks 3.1 and 3.2 succeed, the AIoT computes a pseudonym based on its identifier and the received challenge value. This can be implemented by means of a lightweight function, e.g., a keyed hash function.

In step 4, the AIoT device sends its pseudonym in a response message (e.g., an inventory response message).

In step5, based on the configuration received in step 1, the reader UE/gNB aggregates the response message(s) received, until the time T1 runs out and/or other forwarding criteria (e.g., all device responses are received) are met.

In step 6, the reader UE/gNB forwards the response(s) to the AIoT MF.

6.31.3 	Evaluation

This solution addresses KI#3: Privacy by protecting AIoT device identifiers, in scenarios where a specific device or devices belonging to a group is/are inventoried.

This solution describes a lightweight solution for privacy protection of AIoT device(s) identity(ies).

The solution incorporates a challenge value generated by the AIoT management function (e.g., NF/AF) using device or group specific credentials and sent together with the device or group identifier in the inventory request.

An AIoT device needs to verify the identifier(s) and challenge value received in the inventory request to check whether it is concerned with the inventory request, and only if the verification is successful does it generate a pseudonym and sends a response。

*** End of 2nd Change ***