3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250111

# Online, Electronic meeting, 13 -16 January 2025

Source:	Beijing Xiaomi Mobile Software

Title:	Pseudo-CR on Conclusion for KI#5 in TR 33.713

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.5.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This pCR proposes to conclude KI#5 for FS_AIOT_SEC in TR 33.713.

Proposed Changes

* * * First Change * * * *

7.X	Conclusions for Key issue #5: Authentication in Ambient IoT service

For third party managed AIoT devices, authentication is performed over the application layer.

For operator managed AIoT devices, the following principles are taken as the conclusion:

- The root key is securely stored in the NVM of the AIoT device and in the device subscription data in the 5GC.

- A dedicated AIoTF authentication function is used to authenticate the AIoT device and calculate the session key, which can be collocated with AIoTF.

- The authentication is performed during the Inventory procedure.

Step A: The AIoTF provides a random number in the inventory request message.

Step B: The AIoT device ID and optional RES are included in the inventory response message.

- A specific session key is generated for each AIoT device from its root key.

* * * End of Changes * * * *