3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250128

# Online, Electronic meeting, 13 -16 January 2025

Source:	Ericsson

Title:	Conclusion for KI#1

Document for:	Approval

Agenda item:	5.16

Spec:	3GPP TR 33.766

Version:	0.5.0

Work Item:	FS_Energy_Sec

Comments

For addressing user consent aspects in KI#1, it is proposed to conclude KI#1 by stating that Annex V in TS 33.501 [2] is reused and that the Energy Information Function (EIF) introduced by SA2 is deemed to be the enforcement point.

Furthermore, the solutions for KI#1 does not introduce any new mechanisms for energy related information collection as the solutions reuse existing protection principles. Therefore, this contribution proposes to conclude KI#1 by stating that there is no normative work required and removes all Editor's Notes in KI#1.

Proposed Changes

* * * First Change * * * *

# 5	Key issues

Editor’s Note: This clause contains all the key issues identified during the study.

## 5.1	Key Issue #1: Security aspects of collecting energy related information.

### 5.1.1	Key issue details

TR 23.700-66 [2] studies the collection of energy related information for potential exposure.

The integrity and confidentiality of the data collected is of importance to produce correct analytics metrics.

Another aspect of collecting energy related information is the granularity at which it’s collected. Further information on the granularities can be found in TR 23.700-66 [2].

The key issue aims to address the security issues, ensuring integrity and confidentiality of the energy related information collected.

There is, depending on the use case, a need to consider towards compliance to regional legislation for collecting energy related information.

### 5.1.2	Security threats

Lack of confidentiality, integrity, and replay protection in collecting energy related information can lead to disclosure of the information and tampering of the information.

### 5.1.3	Potential security requirements

The data in transit shall support confidentiality, integrity, and replay protection.

* * * Next Change * * * *

# 7	Conclusions

## 7.1	Key Issue #1: Security aspects of collecting energy related information.

It is concluded that Annex V in TS 33.501 [X] is reused for user consent and that the Energy Information Function (EIF) is deemed to be the enforcement point. Furthermore, it is concluded that no normative work is required for this key issue.

* * * End of Changes * * * *