3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250095

# Online, Electronic meeting, 13 -16 January 2025

Source:	Samsung

Title:	Evaluation to solution#38

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.5.0

Work Item:	FS_Ambient_IoT_Sec

Comments

In the solution detail, the following Editors' Notes are captured:

Editor’s Note: Impact of protected ID on paging occasion is FFS.

Editor's Note: It is FFS how to prevent power and resource exhaustion in the AIoT devices when all of devices in the paging area always have to decrypt the Device ID.

Evaluation part is updated by resolving the above Editor's Notes.

Proposed Changes

* * * First Change * * * *

### 6.38.2	Solution details

Figure 6.38.2-1 Security procedure for AIoT

0. AIoT ID and a key (K) are provisioned to the AIoT device and AF. AF generates KAIoT from K and RANDAF.

1. AF sends AIoT service request message to AIoTF (AIoT Function). This message may be sent via NEF. AIoT service request message includes command, AIoT ID, RANDAF, and KAIoT.

2. AIoTF generates RANDAIoTF. AIoTF derives encryption key (Kenc) and integrity key (Kint), and then encrypts AIoT ID using RANDAIoTF and Kenc.3. AIoTF sends AIoT Service Request including RANDAIoTF, RANDAF, encrypted AIoT ID, and command.

NOTE: If necessary, command may be encrypted using Kenc.

Editor’s Note: Possibility of replay and spoofing attack at step 3 is FFS.

Editor’s Note: How AIoTF knows which AIoT device to send the request is FFS.

4. The AIoT device derives KAIoT from K and RANDAF. The AIoT device derives Kenc and Kint from KAIoT, before decrypting the encrypted AIoT ID. If the decrypted ID is same as the AIoT ID, the AIoT device processes the command.

5. AIoT device responds with AIoT service response. The message includes encrypted AIoT ID, command response, and MAC. MAC is generated from Kint, RANDAIoTF, encrypted ID, and command response.

NOTE: If necessary, command response may be encrypted using Kenc.

Editor’s Note: Possibility of replay and spoofing attack at step 5 is FFS.

6. After AIoTF finds Kenc and Kint from the encrypted AIoT ID, AIoTF checks the authenticity of the message by verifying the MAC. If the verification is successful, AIoTF decrypts the encrypted IE(s) and proceeds to step 7. If the verification fails, AIoTF does not allow the AIoT device to use the network provided by the operator.

7. AIoTF sends AIoT service response to AF. This message includes AIoT ID and command response.

For A-IoT device paging functionality, it is understood that the legacy paging message, legacy paging occasion and legacy DRX from NR are not supported (See TS 38.300 [R2-3] for references for any legacy NR functionality). From RAN2 perspective, it is assumed that the A-IoT device can receive as long as there is enough energy

### 6.38.3	Evaluation

This solution addresses the requirement of Key Issue #3 by using encrypted ID and Key Issue #5 by performing one-way authentication.

Due to the encrypted ID in the paging message, AIoT devices in the area need to try the decryption to check whether the encrypted ID is its own ID. So, AIoT devices in the area may suffer from power and resource exhaustion.

Editor's Note: Further evaluation is FFS.

* * * * End of Changes * * * *