3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250119

# Online, Electronic meeting, 13 -16 January 2025

Source:	Xiaomi

Title:	Pseudo-CR on TR 33.700-22 KI#2 conclusion update

Document for:	Approval

Agenda item:	5.19

Spec:	3GPP TR 33.700-22

Version:		0.3.0

Work Item:	FS_CAPIF_Ph3-sec

Comments

This contribution updates conclusion for KI#2.

Proposed Changes

* * * First Change * * * *

7.2	Conclusion for KI #2: CAPIF interconnection security

7.2.1 	Conclusion for CAPIF 6/6e security

It is concluded that for CAPIF-6 and CAPIF-6e reference points, same security mechanisms specified in clauses 6.6 and 6.10 of TS 33.122 [4] for CAPIF-3/4/5 and CAPIF-3e/4e/5e reference points will be used, respectively.

7.2.2 	Conclusion for security method negotiation

Assume CCF-B is connected with the target AEF. API invoker onboards CCF-A.

Security method negotiation defined in clause 6.3.1.2 of TS 33.122 [4] can be reused with the following modifications.

- Target AEF provides the supported security mechanisms to CCF-B.

- If CCF-A finds the target AEF is discovered by CCF-B, CCF-A sends the common security methods that are supported by both CCF-A and the API invoker to CCF-B.

- CCF-B selects a security method to be used over CAPIF-2/2e reference point for each requested AEF, taking into account the information from CCF-A and AEF capabilities.

- CCF-B sends the selected method to the API invoker via the CCF-A.

7.2.3 	Conclusion for API invoker authentication and authorization mechanism

Assume CCF-B is connected with the target AEF. API invoker onboards CCF-A.

Authentication and authorization mechanism defined in clause 6.5.2.1 and clause 6.5.6.2 of TS 33.122 [4] can be reused with the following modifications.

- In authentication initiation request, the API invoker is required to send CCF-A ID to the AEF.

- To get AEFPSK/root CA certificate from CCF-B, the AEF sends AEF ID, API invoker ID and CCF-A ID to CCF-B. The CCF-A ID is used by CCF-B to identify CCF-A and send the request to CCF-A.

- To request AEFPSK/root CA certificate from CCF-A, CCF-B sends AEF ID and API invoker ID to the CCF-A. The AEF ID and API invoker ID are used by CCF-A to retrieve security information related to the specific AEF and API invoker.

Authentication and authorization mechanism defined in clause 6.5.2.3 and clause 6.5.3 of TS 33.122 [4] can be reused with the following modifications.

- If CCF-A finds that the expected service/service operation/service API cannot be provided by the AEF in its domain and finds that the expected service/service operation matches the discovered service API from CCA-B, CCF-A sends the access token request to the CCF-B.

- If CCF-A finds that the expected service/service operation/service API is previously published by CCF-B, the CCF-A sends the access token request to the CCF-B.

- CCF-B generates the token for the API invoker and sends the token to the API invoker via the CCF-A.

Editor’s note: Further conclusions are FFS.

* * * End of Change * * * *