3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250149

# Online, Electronic meeting, 13 -16 January 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Update sol#7 on authorize avatar by metaverse service provider

Document for:	Approval

Agenda item:	5.18

Spec:	3GPP TR 33.721

Version:	0.5.0

Work Item:	FS_Metaverse_Sec

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

The contribution proposed to update solution# 7 of the TR on authorize avatar by metaverse service provider, to address the ENs, and add evaluation for the solution.

As user(s) behind the UE is not in the scope 3GPP R19, also user identity and VAL user identify definition in 23.434 still need to be clarified. The solution assumes only one VAL user is supported for a VAL UE, which is corresponding to the subscriber of the UE and the VAL user id can be mapped to the VAL UE Id or GPSI.

Proposed Changes

* * * First Change * * * *

<Proposed change in revision marks>

## 6.7	Solution #7: authorize avatar by metaverse service provider

### 6.7.1	Introduction

This solution addresses Key Issue #4 on Authentication and authorization of digital representation.

Avatar and digital asset support, including digital asset avatar management and discovery, is discussed in Key issue #3 in TR 23.700-21[2]. The corresponding requirements were defined in clause 5.4 of the TR 23.700-21[2] which include, for example, subject to operator policy, regulatory requirements and user consent, the metaverse enablement service shall provide digital asset management mechanisms as follows:

-	to create, update, retrieve, delete and discover digital assets securely.

-	to manage associations between digital assets and user identifiers.

-	to allow an authorized third party to manage digital asset(s) associated with a user.

Correspondingly, KI#4 on Authentication and authorization of digital representation was described in TR 33.721 which requires 5G system support authenticating a digital representation to represent a user in mobile metaverse services and authorizing the VAL user to use the digital representation (avatar) in mobile metaverse service.

Architecture, procedures and information flows for digital asset service to support mobile metaverse services are specified in TS 23.438 [8]. Users can be associated with one or more digital assets like Avatars, software licenses, files, etc. Applications like mobile metaverse services can utilize the digital assets related to users, and the users can benefit from having the use of their digital assets between the various metaverse applications/platforms in an interoperable way.

In the current release of this specification, the digital asset type is limited to digital avatar.

This solution aims to support metaverse service provider to authorize the VAL user to use the avatar when the users have the use of their avatars between the various metaverse service providers.

### 6.7.2	Solution details

Figure 6.7.2-1 Procedure to authorize usage of avatar in metaverse service

Precondition:

- Avatar is created and stored in digital asset server which included authorization information provisioned by the owner of the digital asset as part of DA profile, which includes owner list, allowed user list, allowed application list, spatial conditions, expiry time, etc., refer to clause 7.2 of TS 23.438 [8] for the detail authorization information in DA profile.

- Digital asset (DA) client of the VAL UE retrieves avatar from the DA server.

- VAL client of the VAL UE retrieves avatar from the DA client inside UE via DA-C interface which is out of scope of 3GPP.

Note: Assume only one VAL user is supported for a VAL UE, which is corresponding to the subscriber of the UE and the VAL user id can be mapped to the VAL UE Id or GPSI.

Procedure:

1. The VAL client , on behalf of the VAL user, sends metaverse service request to a metaverse service provider including VAL user id, avatar and avatar id.

2. The metaverse service provider sends request to DA server to check if the VAL client on behalf of the VAL user is allowed to use the avatar, the request includes avatar/DA id, VAL user id , application id of the metaverse service provider, location of the VAL client or metaverse service provider, usage time.

3. The DA server retrieves DA profile based on DA id, and checks if the VAL user id is in the allowed user id list, the application id(s) is in the allowed application list, the location is matched spatial conditions, and the usage time is before expiry time of the DA/avatar.

4. The DA server returns the checking result to the metaverse service provider.

5. Metaverse service provider proceeds with the metaverse service request, e.g. show the avatar in spatial anchor/map, and sends response to the DA client.

### 6.7.3	Evaluation

The solution addresses requirements of Key issue #4 to support authorizing the VAL user to use the digital representation (avatar) in mobile metaverse service.

SEAL server, e.g. DA server, needs to enhance to support authorization request from other VAL/SEAL server such as spatial anchor/map server.

The solution assumes only one VAL user is supported for a VAL UE, which is corresponding to the subscriber of the UE and the VAL user id can be mapped to the VAL UE Id or GPSI.

* * * End of Changes * * * *