3GPP TSG-SA3 Meeting #119	S3-244791

Orlando, US  11 - 15 November 2024

# Source:	Nokia

# Title:	Updates to conclusions for split MME architecture

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

It is required to approve this contribution for updates to conclusions related to split MME architecture.

# 2	References

[1]	3GPP TR 33.794 “Study on enablers for Zero Trust Security”

# 3	Rationale

The contribution is intended to enhance the conclusions for split MME architecture.

# 4	Detailed proposal

# * * * * First change* * * *

## 7.2	Conclusions for Key Issue #1: Security protection in Store and Forward Satellite Operation

For the HSS on the satellite deployment, the following conclusions are made:

The normal security procedures are used as security can be established using legacy security procedures (e.g. AKA, NAS security mode command, etc.) using the security credentials in the HSS onboard the satellite;

As an option for the operator, IOPS-based keying which is captured in the informative Annex F in TS 33.401 [3] can be used to limit the impact of exposing the long-term key in the HSS in the satellite; and

NOTE: Any related enhancements of IOPS-based keying agreed during the normative phase are to be captured in the informative Annex F in TS 33.401 [3].

The security of communications between the proxies on satellite and ground is out of 3GPP scope.

The following principles are agreed for supporting Store and Forward operation with a split MME architecture:

-	Mutual authentication between the UE and the 3GPP network in S&F operations may require the involvement of more than one MME on-board a satellite, in which case the ground network is responsible for the selection and provisioning of MMEs on-board the same, or another satellite with the necessary information to perform or finish a mutual authentication procedure.

-	It has to be ensured that the latest NAS security context of the UE is available at the MME on-board which processes the NAS security;

-	The security of communications between the proxies on satellite and ground is out of 3GPP scope.

-	Enhancements to provide protection against DoS attacks before a security context is established are to be determined during normative phase based on the solution(s) captured in this document.

NOTE: Solutions proposing changes which impact multiple entities and/or having major procedural impacts may be included in informative manner as possible implementation options.

- Changes required for timer related aspects for S&F mode (e.g., backoff timer, NAS timer updates, UE wait timer, etc.) are to be determined during the normative phase based on relevant solutions.

Editor’s Note: Further conclusions are FFS.

# * * * * End of changes * * * *