3GPP TSG-SA3 Meeting #119	S3-244831

Orlando, USA  11 - 15 November 2024

# Source:	OPPO, Xidian

# Title:	Updating Solution #7

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

Approve the pCR to TR 33.713

2	References

[x]	Novel Authentication Protocols Tailored for Ambient IoT Devices in 3GPP 5G Networks (https://web3.arxiv.org/pdf/2404.02425)

[y]	TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[z]	TS 33.102 3G security; Security architecture

[xx]	TS 35.205 Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: General

[yy]	R2-2406202 RAN2#126 Meeting Report

3	Rationale

This contribution proposes updates and evaluation for AIoT security solution #7.

Proposed change 1: According to the latest conslution of SA2 23700, for topology 1

-	Communicate with BS Reader (i.e. AIoT RAN), either directly or via an AMF, to trigger e.g., inventory request and command request).

Thus, relative Notes are added for step 6 and step 12.

Proposed change 2: To align with SA2’s conclusion of AIoT Identifier and Identification Management:

-	An Ambient IoT Device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. The Identifier is used to identify Ambient IoT Device and locate the corresponding authentication server.

It is added “The security credential(s) includes AIoT device ID and the root key, are stored in the UDM or the authentication server of the 3rd party”.

Proposed change 3: According to the lasted agreement of RAN2, there is meand to avoid duplicate response from the device to a reader.  Thus, the  SEQNO is not needed and the related ENs can be deleted.

Editor's Note:	The impact of removing the use of SEQNO (e.g., replay against device or network) from AKA is FFS

Agreements

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

Proposed change 4: AK in this solution is not used to protect SEQNO, so the related ENs can be deleted. AK is used to make the input parameters different for MAC and RES in this solution.

Editor's Note:	Anonymity key (AK) is used to protect SEQNO in AKA. If SEQNO is not used, the necessity of using AK to compute MAC and XRES is FFS.

Proposed change 5: This solution is for “inventory and command” case, so before command is sent, the AIoT NF triggers the authentication. Thus, related text was added to step 1, and the following EN is deleted.

Editor's Note:	When the AIoT NF triggers the authentication is FFS.

Proposed change 6: It is added “Function 1 and Function 2 can be f1 and f2”, so the related ENs can be deleted.

Editor's Note:	Further evaluation of the usage of the crypto primitives is FFS.

Editor's Note:	The security level of MAC/RES calculation compare to 5G-AKA is FFS.

Proposed change 7: It is added “Note: The storage of long-term root key is based on platform implementation.” and “Note: The storage of authentication parameters is based on platform implementation” in the solution, so the related ENs can be deleted.

Editor's Note:	It is FFS if the shared key needs confidentiality/integrity protection against physical attack in the device

Editor's Note:	The intermediate authentication parameter storage in AIoT device is FFS.

Proposed evaluation: Considering the power constrain of AIoT device (1uw or few humdres uw), power comsumption of solution must be taken acoount in to the solution evaluation.

For this document, since W=P*T, it compares the running time of this lightweight authentication solution and 5G AKA using the same hardware. In this way we compare the power comsumption of this solution compared to 5G AKA.

Using same hardware, the time of running this solution compared to 5G AKA is 1.90%.

4	Detailed proposal

SA3 is suggested to approve the following pCR.

*** Start of 1st Change ***

6.7			Solution #7: Lightweight AIoT Authentication solution

6.7.1			Introduction

The assumption of this solution is AIoT device and network sharing a long-term root key K[x].

Note: The storage of long-term root key is based on platform implementation.

MAC/XMAC is used for device authenticating network. MAC is calculated by network side with the K, XMAC is calculated by AIoT device side with the K.

Optionally, RES/XRES is used for network authenticating device. XRES is calculated by network side with the K, RES is calculated by device side with the K.

To provide the freshness of the authentication vectors, RAND is also used to calculate MAC/XMAC and optionally RES/XRES.

6.7.2			Details

MAC/XMAC is used for device authenticating network. MAC is calculated by network side with the K, XMAC is calculated by AIoT device side with the K.

Optionally, RES/XRES is used for network authenticating device. XRES is calculated by network side with the K, RES is calculated by device side with the K.

To provide the freshness of the authentication vectors, RAND is also used to calculate MAC/XMAC and optionally RES/XRES.

Similar with 5G-AKA, and based on RAN2 agreement for “inventory and command” case[zz], AIoT authentication solution has the following steps:

Figure 6.7.2-1 AIoT Authentication based on 5G-AKA

- AIoT NF sends authentication vector request to UDM/ARPF or authentication server of the 3rd party for AIoT device. The AIoT NF triggers the authentication before sends the Command to the AIoT device.

The security credential(s) includes AIoT device ID and the root key, are stored in the UDM or the authentication server of the 3rd party

- UDM/ARPF calculates MAC with K (e.g. the root key of AIoT device) and RAND.

- Optionally, UDM/ARPF calculates XRES with K and RAND, if network wants to authenticate AIoT device.

- UDM/ARPF sends RAND, MAC, device ID and optionally XRES to AIoT Authentication Function.

- AIoT Authentication Function sends Authentication Request including RAND, MAC, device ID to AIoT NF.

- AIoT NF sends Authentication Request including RAND, MAC, device ID to Reader.

NOTE: According to SA2 architecture options, if it’s indirect communication between a BS Reader and an AIoTF via an AMF, then message 6 is transfer via AMF.

- Reader sends Authentication Request including RAND and MAC to AIoT device. Authentication Request is carried by Step C in the “inventory and command” case.

- AIoT device calculates XMAC with RAND and K.

- Optionally, AIoT device calculates RES with K and RAND.

- AIoT device verifies XMAC=MAC, then the network authentication is successful. If there is command, only after successful verification, the command can be proceeded.

Note: The storage of authentication parameters is based on platform implementation.

- AIoT device sends Authentication Reponses to Reader, this message optionally including RES if network wants to authenticate AIoT device. Authentication Reponses is carried by Step D in the “inventory and command” case.

- Reader sends Authentication Reponses to AIoT NF.

Note: According to SA2 architecture options, if it’s indirect communication between a BS Reader and an AIoTF via an AMF, then message 12 is transfer via AMF.

- Optionally, AIoT NF sends Uplink Authentication Request including RES to AIoT Authentication Function.

- Optionally, AIoT Authentication Function verifies XRES=RES, then the AIoT device Authentication is successful.

As an example, the generation of MAC/XMAC and RES/XRES is described as below. Function 1 and Function 2 can be f1 and f2.

Figure 6.X.2-2 The generation of MAC and XRES at network side.

Figure 6.X.2-3 The generation of MAC and XRES at AIoT device side.

6.7.3			Evaluation

The power constrain of AIoT device is described in TR 38.769[y].

For 5G AKA, in order to derive authentication vectors at the AIoT device side, f1~f5 and HMAC for Res* are needed.

For lightweight AIoT authentication solution, in order to derive MAC/XRES at the network side, only 2 functions are needed. If lightweight functions such as AES is used for these functions [xx], it will further reduce the power consumption.

Using the same hardware[x], the running time of this lightweight authentication solution and 5G AKA are compared as follows:

Time of running the 5G AKA is

- 3515us *5+ 352417 us = 369992us = 369.992ms = 0.369992s

since

f1-f5 (CK, IK, AK, MAC, XRES) = 3515*5us

HMAC→RES* = 352417us

Time of running this solution is

- 3515us *2 = 7030us = 7.030ms = 0.00703s

since

f1, f2 (XMAC, RES) = 3515*2us

Based on same platform, the time of running this solution compared to 5G AKA is 1.9%:

- 0.00703s/0.3699s = 1.9%

Conclusion of this evaluation: this solution has significant power reduction compared to 5G AKA and it’s suitable the AIoT device.

*** End of 1st Change ***

*** Start of 2nd Change ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service Requirements for ambient power-enabled IoT".

[3]	RP-234058, RAN New SID for Study on Solution for Ambient IoT in NR.

[4]	3GPP TR 23-700-13: "Study on Architecture Support of Ambient power-enabled Internet of Things".

[5]					3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[6]					R2-2406202 RAN2#126 Meeting Report

[7]	RFC 4739: "Multiple Authentication Exchanges in the Internet Key Exchange (IKEv2) Protocol".

[8]	3GPP TR 38.848: "Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN".

[x]	Novel Authentication Protocols Tailored for Ambient IoT Devices in 3GPP 5G Networks (https://web3.arxiv.org/pdf/2404.02425)

[y]	TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[z]	TS 33.102 3G security; Security architecture

[xx]	TS 35.205 Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: General

[yy]	R2-2406202 RAN2#126 Meeting Report

*** End of Changes ***