3GPP TSG-SA3 Meeting #119	S3-244987

Orlando, US, 11 -15 November 2024

# Source:	Ericsson

# Title:	New conclusion for KI#1

# Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

It is proposed to approve the following conclusion to 33.784 [1]

# 2	References

[1]	TR 33.784 V0.4.0 Study on security aspects of Core Network Enhanced Support for AIML

[2]	S2-2411257, Addressing Editor´s Note: Procedure for Data Collection for AI/ML based positioning from NG-RAN

[3]	TS 23.288 V18.7.0 Architecture enhancements for 5G System (5GS) to support network data analytics services

[4]	C4-244355, LMF as Consumer of User Consent Subscription Data

# 3	Rationale

This contribution proposes a conclusion to KI#1 "Security aspects on enhancements to LCS to support AIML" to address the EN in approved SA2 CR S2-2411257 [2] related to user consent aspects for LMF-based AI/ML Model positioning.

The existing user consent mechanism in the UDM for data collection and model training purpose has been specified in TS 23.288 [3] for analytics and model training service e.g. when the NF Service Consumer is an NWDAF. In context of service for enhancements to LCS to support AIML, both LMF and NDWAF can perform data collection and Model training for LCS. Hence, the LMF can utilize the same procedure as the NWDAF to fetch user consent from the UDM, which covers both data collection and model training. In this regard, the agreed CT4 CR C4-244355 [4] implements LMF as a NF Service Consumer for Nudm Services and for User Consent Subscription Data to allow the LMF to retrieve user consent for data collection and model training purposes from the UDM.

Based on above, it is proposed to conclude KI#1 by stating that LMF utilizes UDM to retrieve user consent for the purpose of data collection and model training.

# 4	Detailed proposal

*** Start Change ***

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 7.1	Conclusion on Key Issue #1

For Key Issue#1, it is recommended to use the following principles as the baseline for enhancements to LCS to support AIML:

-	The LMF shall register its vendor ID in the NRF, allowing the NRF to authorize the LMF to retrieve model.

Editor's Note: Whether interoperability indicator or new indicator is used needs to be coordinated with SA2.

Regarding user consent aspect for LMF-based AI/ML model positioning, it is concluded that Annex V of TS 33.501 will be re-used as done in TS 23.288; thus, there is no normative work required.

*** End Change ***