3GPP TSG-SA3 Meeting #119	S3-244991

Orlando, US, 11 -15 November 2024

# Source:	Ericsson

# Title:	Conclusion update for KI#1

# Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

It is proposed to approve the following updates to TR 33.784 [1]

# 2	References

[1]	TR 33.784 V0.4.0 Study on security aspects of Core Network Enhanced Support for AIML

[2]	S2-2410997, Addressing the EN about model provision for AI positioning

# 3	Rationale

The conclusion for KI#1 contains the following Editor's Note:

Editor's Note: Whether interoperability indicator or new indicator is used needs to be coordinated with SA2.

As agreed in SA2 CR S2-2410997 [2], a new indication is used to indicate support of model training and ML model retrieval for LMF-based AI/ML positioning. Therefore, it is proposed to remove the above Editor's Note by adding a new bullet stating that LMF utilizes a new indication to indicate retrieval of ML model for LMF-based AI/ML positioning.

# 4	Detailed proposal

*** Start Change ***

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 7.1	Conclusion on Key Issue #1

For Key Issue#1, it is recommended to use the following principles as the baseline for enhancements to LCS to support AIML:

-	The LMF shall register its vendor ID in the NRF, allowing the NRF to authorize the LMF to retrieve model.

-	The LMF utilizes a new indication to indicate retrieval of AI/ML model for LMF-based AI/ML positioning. Therefore, the LMF may include the indicator in the access token request and the NRF may include the same indicator in the access token.

*** End Change ***