3GPP TSG-SA3 Meeting #119	S3-245249

Orlando, US, 11 -15 November 2024	revision of S3-245060

# Source:	Ericsson

# Title:	Update to Solution #25 and evaluation

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to add the following update to the TR 33.700-22.

# 2	References

[1]	3GPP TR 33.700-22: "Study on security aspects of CAPIF Phase3".

# 3	Rationale

This document provides an update to Solution #25 and evaluates the solution.

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.700-22 [1].

***START OF THE CHANGE***

## 6.25	Solution #25: Backend based solution for UE-deployed API invoker accessing resources not owned by that UE

### 6.25.1 	Introduction

This solution addresses the requirements identified in key Issue #3 (Authorizing API invoker on one UE accessing resources related to another UE).

In the real word application deployments, it is very common practice to have a backend server in addition to the application clients running/installed on end point devices such as mobile phones. For example, when the use case of gaming application is considered, even if the game application client instances are running on different devices and communicating with each other directly, there is always a backend server serving these clients. Thus, utilization of backend server can bring some benefits in terms of handling access permission and user identifiers. Taking these benefits into account, this solution proposes that when an application client instance running on a UE wants to access resources related to another UE which hosts another application client instance, the requester client can send the request to the backend server and then the backend server can consume the related APIs and returns the result back to the requester client. From 3GPP point of view, only permission check whether the backend server as AF is allowed to access the resources of the UE is needed. The remaining permission check such as which users’ resource can be accessed by which users can be handled in the application layer out of 3GPP scope.

### 6.25.2	Solution details

Figure 6.24.2-1 illustrates the high-level architecture.

Figure 6.25.2-1: High level architecture

High-level steps of the solution are presented in Figure 6.25.2-2 and explained below.

Figure 6.25.2-2: High-level procedure flow of the solution

1.	The application instance running on UE 1 sends a request to the backend server to access resources related to user 2. The communication between the application instance and backend server and its security are out of scope of the solution.

2.	The backend server identifies the UE ID of UE 2 related to the request. How to identify the UE ID is out of scope of the solution. Also, the backend server handles the resource owner authorization in the application layer (e.g., checks if user 1 can access resources related to user 2, gets permission from user 1). Application layer resource owner authorization handling is out of scope of the solution. The backend server acting the role of AF obtains an access token to access resources related to UE 2 by executing authentication and authorization the steps specified in TS 33.122 [4] if there is no valid access token. In the access token request, the AF can also send information about the UE 1 and the CCF can use that information in the authorization decision.

3.	The backend server acting the role of AF invokes the API of the AEF with the access token.

4.	After successful authentication of the backend server (i.e., AF) and verification of the access token, the AEF processes the API invocation and returns the result.

5.	The backend server can return the result.

### 6.25.3	Evaluation

This solution addresses KI#3 by proposing invocation of the APIs by the back-end server assuming the role of AF, instead of direct API invocation by the application instance running on the UE. With that proposal, the solution have no impact on CAPIF except the impact on the API invoker side.

***END OF THE CHANGE***