3GPP TSG-SA3 Meeting #119	     S3-245368

Orlando, Florida, 11 – 15 Nov 2024

Source:	Google

Title:	Conclusion for KI#5 (Certificate Renewal)

Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approve the pCR to TR 33.776

# 2	References

[1]	3GPP TR 33.776: “Study of ACME for Automated Certificate Management in SBA”

[2]	IETF RFC 8555: “Automatic Certificate Management Environment (ACME)”, 2019

[3]	3GPP TS 33.310: “Network Domain Security (NDS); Authentication Framework (AF)”

# 3	Rationale

This contribution proposes a conclusion to KI#5 in TR 33.776.

# ******************   START OF CHANGES - ALL NEW TEXT **********************

## 7.x. 	KI#5: Certificate renewal

### 7.x.1	Analysis

This key issue is addressed by two solutions, namely Solution #4 (Reuse solution about policy-based certificate renewal), which proposes to reuse the solution adopted in the study of FS_ACM during Release 18, and Solution #9 (Using ACME protocol for certificate renewal). Solution #4 depends on the preconfigured policy. Similarly, in solution #9 the NF initiates certificate renewal procedures based on the trigger from pre-configured certificate renewal policies. While both solutions intend to adopt policy-based certificate renewal principles, solution #9 also includes the procedure by which certificate renewal is carried out. Solution #9 involves two different methods of ACME client authorization for certificate renewal: the ‘challenge-response’ process and the ‘pre-authorization’ process, wherein pre-authorization process is optional.

### 7.x.2 Conclusion

It is recommended to include certificate renewal procedure based on Solution #4 and the ‘challenge-response’ process in Solution #9 (excluding “pre-authorization” process).

******************    END OF CHANGES   ************************