3GPP TSG-SA3 Meeting #119	S3-244988

Orlando, US, 11 -15 November 2024

# Source:	Ericsson

# Title:	Resolution of ENs and updates in Solution#16

# Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

It is proposed to approve the following updates to TR 33.784 [1].

# 2	References

[1]	TR 33.784 V.0.4.0 Study on security aspects of Core Network Enhanced Support for AIML

[2]	S2-2410997, Addressing the EN about model provision for AI positioning

# 3	Rationale

The solution details of Solution #16 (LMF authorization for AI/ML model retrieval from NWDAF containing MTLF) contains the following Editor's Notes:

Editor's Note: When the service consumer is LMF, whether to use a specific analytics ID or a new indication to indicate to retrieve ML model for LMF based AI/ML positioning needs to be aligned with SA2.

Additionally, the evaluation of Solution #16 contains the following Editor's Note:

Editor's Note: This solution is dependent on SA2 outcome as whether to utilize an analytics ID or a different indication when the consumer is LMF, not an NWDAF, needs to be aligned with SA2.

In the agreed SA2 CR S2-2410997 [2], it has been decided that that a new indication is utilized to indicate support of model training and ML model retrieval for LMF-based AI/ML positioning. Moreover, it is stated in a NOTE that the implementation of this new indication is up to stage 3 to implement. In this regard, the above two Editor's Notes can be removed.

Editor's Note: Whether analytics ID/new indicator is needed in the token request is FFS.

Moreover, the above Editor's Note in solution details of Solution #16 is removed with the following reasoning: the new indication is needed in the access token request for granular service authorization at the NWDAF (like analytics ID usage in HFL). Having the new indication in the access token, thereby in the token claim, allows the NWDAF to authorize the LMF only for the service that is tied to the new indication in the token request, which in this case is for AI/ML model retrieval from NWDAF.

Lastly, the solution figure and the steps related to analytics ID/new indication is updated to remove analytics ID and only keep new indication.

# 4	Detailed proposal

*** Start Change ***

## 6.16	Solution #16:	LMF authorization for AI/ML model retrieval from NWDAF containing MTLF

### 6.16.1	Introduction

The proposed solution addresses the security requirement of key issue #1. The security requirement states: "If AI model training entity and AI model consumer are different, 5GS shall support authentication and authorization of AIML model or training data retrieval between LMF and NWDAF." In addition, the requirement has a NOTE stating: "The procedures defined in TS 33.501 [5] Annex X.10 Security for AI/ML model storage and sharing needs to be taken into account.". The proposed solution reuses the principles in Annex X.10 with the difference that LMF is the entity retrieving the model from NWDAF containing MTLF, and that model storage in the ADRF is not applicable.

### 6.16.2	Solution details

The solution is shown in Figure 6.16.2-1.

Figure 6.16.2-1: LMF authorization for AI/ML model retrieval from NWDAF

The solution includes the following steps:

0a.	NF Service producer, i.e. NWDAF containing MTLF, registers its NF profile in the NRF with ML model interoperability indicator per new indication.

0b.	NF service consumer, i.e. LMF, registers at the NRF including its Vendor ID and a new indication.

1.	NF Service consumer, i.e. LMF, performs Nnrf_NFDiscovery_Request operation

2a.	NF Service consumer, i.e. LMF, requests an access token from the NRF using the Nnrf_AccessToken_Get request operation. The token request message contains, besides the parameters described in clause 13.4.1.1.2, the Vendor ID of the NF Service consumer and the new indication.

2b.	NRF checks whether the NF service consumer is authorized to access the requested service in NWDAF containing MTLF. The NRF verifies that the NF Consumer's Vendor ID is included in the NF Service Producer's interoperability indicator for the new indication. If the authorization is successful, the NRF grants the token including the new indication as specified in the clause 13.4.1

3.	NF Service consumer performs Nnwdaf_MLModelProvision Request (new indication, Vendor ID and token) service operation at the NWDAF containing MTLF to retrieve ML models for the new indication.

4.	The NF Service Producer authenticates the NF Service Consumer and verifies the access token as specified in the clause 13.4.1.1.2 and ensures that the new indication is included in the access token. If verification is successful, NWDAF containing MTLF determines the ML model to be shared for the requested new indication.

5.	NWDAF containing MTLF sends Nnwdaf_MLModelProvision Response to the NF Service Consumer with Model ID, the address of the determined ML model.

### 6.16.3	Evaluation

The proposed solution reuses the defined principles in Annex X.10 of TS 33.501 to authorize LMF to retrieve AI/ML from NWDAF. In this solution, the LMF is the NF Service consumer, and the AI/ML model is retrieved directly from the NWDAF, not from the ADRF; thus, the AI/ML model storage and sharing in ADRF is not included in the solution.

*** End Change ***