3GPP TSG SA WG3 (Security) Adho Meeting on FS_NSA	S3-161511

27-29 September 2016 San Diego, USA		revision of S3-161319

Source:	ORANGE, Telecom Italia, Gemalto, Deutsche Telekom, China Mobile, Oberthur, Giesecke&Devrient, AT&T

Title:	Additional Security requirements for Next Generation System User Equipment

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	Architecture and Security For Next Generation System / Rel-14

Abstract of the contribution:

It proposes additional security requirements regarding the storage of credentials and sensitive data for Next Generation System.

1. Discussion

The security of 3GPP Next Generation System is crucial.

This contribution proposes additional security requirements to cover the threats described in clause 5.5.3.1.2

2. Proposal: pseudo-CR to TR 33.899 on security aspects of Next Generation System.

### 5.5.3	Key issues

#### 5.5.3.1	Key issue #5.1: Secure storage and processing of credentials and identities

##### 5.5.3.1.1	Key issue details

The Next Generation System requires the storage of credentials and identities (human and machine) in the User Equipment.

Subscriber (human and machine) credentials and identities allow the network operator to authenticate its subscribers. Subscriber authentication is needed to identify the origin and destination of the communication, to guarantee the Quality of Service and fulfill contractual, legal and regulatory obligations.

##### 5.5.3.1.2	Security threats

An attacker may perform software and/or hardware attacks on the UE in order to extract the subscription credentials and identities that the attacker could then (re)use to clone the subscription.

The extraction of subscriber credentials and identities would have the following threats for the subscriber, e.g.:

-	The attacker could intercept the communications of the subscriber.

-	The attacker could use the cloned subscriber credentials and identities with his/her Mobile Equipment to impersonate the subscriber. For example, the subscriber would be billed for communications he/it is not the originator of.

The extraction of subscriber credentials and identities would have the following threats for the network operator, e.g.:

-	The operator could lose income because:

o	The attacker could use the cloned subscriber credentials and identities with Mobile Equipment and impersonate the subscriber and thereby using another subscribers subscription for his/her own good.

o	The attacker could use the cloned subscriber credentials and identities with both his/her original Mobile Equipment and in other Mobile Equipment for which the attacker does not pay, e.g. in cases,  (where a subscription is intended for use in one single Mobile Equipment at any given time).

o	The attacker could distribute the cloned subscriber credentials and identities to other users such that an unlimited data plan could be shared between several users across many User Equipments.

Editor's Note: Subscriber versus user is for further study.

-	The operator could no longer guarantee the origin and destinations of communications. This could have impacts on the billing, on the quality of service of the network, on the subscriber confidence in the network operator (brand reputation).

-	The attacker could retrieve a secret allowing the remote administration of the subscription parameters which could eventually, in turn, lead to a denial of service attack.

-	The network operator could no longer fulfill contractual, legal and regulatory obligations.

##### 5.5.3.1.3	Potential security requirements

Editor’s Note: the following requirements are ffs.

Within 3GPP Next Generation System:

- All subscription related permanent secrets, used to authenticate the NG-UE to the NextGen 3GPP Networks, shall exclusively be set and managed by the subscription credentials owner.

NOTE: The management operations include read/write and update.

Editor’s Note: the following requirements are ffs.

-	The subscriber (human and machine) credentials and identities shall be stored within the NG-UE  in a secured and tamper resistant hardware entity.

-	It shall be possible to perform a security evaluation / assessment according to the respective security requirements of the entity storing and processing the subscriber credentials and identities.

Editor's Note: The security assurance scheme to be used for evaluation is ffs.