3GPP TSG SA WG3 (Security) Meeting #86bis                                                   S3-170774

27- 31 March, 2017, Busan, Korea

Source:	China Mobile

Title:	pCR to adding new key issue: User location confidentiality

Document for:	Discussion/approval

Agenda Item:	8.4.7

Work Item / Release:	Architecture and Security For Next Generation System / Rel-14

Abstract of the contribution: This pCR identifies the need to mitigate the linkability attack when using the AKA protocol. The related security threats and potential security requirements are given.

Introduction

This contribution identifies that the linkability attack on the AKA protocol is possible due to the different error message sent in case of authentication failure.  Such weakness should be overcome in the Next Gen in order to assure the confidentiality of user location.

Reference

[1]  M. Arapinis etc., New Privacy Issues in Mobile Telephony: Fix and Verification, CCS’12, October 16–18, 2012, Raleigh, North Carolina, USA.

Proposed pCR

***	BEGIN OF THE FIRST CHANGE	***

5.7.3.x 	Key Issue #7.x:  User location confidentiality

5.7.3.x.1 	Key issue details

The AKA (Authentication and Key Agreement) protocol plays a critical role in the security architecture for its twofold functions:   one is mutual authentication between UE and the network, the other is the establishment of the shared session key between UE and the network. EPS AKA is a minor improvement to the UMTS AKA by including the network identifier SNid in generating the intermediate key KASME .  Considering the unified authentication framework, EAP-AKA` might be a candidate authentication protocol used in 5G.   EAP-AKA` works following the EAP framework by using EPS AKA as the authentication method. In case of authentication failure, three protocols ( UMTS-AKA, EPS-AKA, EAP-AKA`) send the error messages from UE to the network， including  MAC failure ( UE fails to verify MAC) or Synch failure ( the sequence number is not within the correct range) .  That would violate user location confidentiality, which is defined as “the property that the presence or the arrival of a user in a certain area cannot be determined by eavesdropping on the radio access link” in TS 33.102.

5.2.3.x.2 	Security threats

As demonstrated in [1], the UMTS-AKA protocol suffers from the linkability attack because two different error messages (MAC failure, Synch failure) in case of authentication failure may be sent from UE to the network. To detect the presence of a UE, an active attacker captures one UE’s legitimate authentication request message containing a pair of (RAND, AUTN) and binds it with that UE. In this attacker doesn’t need to get UE’s IMSI/MSISDN. The attacker replays such authentication request message containing the pair (RAND, AUTN) that has captured before. The adversary can distinguish any mobile station from the one the authentication request was originally sent to according to the type of error message. If the adversary receives a Synch failure message, he/she can ascertain the presence of the UE in a particular area. This breaches the user location confidentiality.  EPS-AKA and EAP-AKA` are subjected to the linkability attack like UMTS-AKA because they inherit the error messages (MAC failure, Synch failure) from UMTS-AKA.

5.2.3.x.3	Potential security requirements

User location shall be kept secret even if active attacks are present.

***	END OF THE FIRST CHANGES	***