3GPP TSG SA WG3 (Security) Meeting #86bis	S3-170817

27-31 March 2017, Busan (Korea)

Source:	NEC

Title:	pCR to TR 33.899: Inter AMF, Intra SMF, Inter NG RAN handover without Xn interface

Document for:              Approval

Agenda Item:	5.1.1

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR proposes a solution for Inter AMF, Intra SMF, Inter NG RAN handover without Xn interface to Security Area #1 Architectural aspects of Next Generation security

#### Introduction

#### This pCR proposes a solution for Inter AMF, Intra SMF, Inter NG RAN handover without Xn interface.

#### Proposal

***** Start of Change *****

#### 5.1.4.x	Solution #1.x Inter AMF, Intra SMF, Inter NG RAN handover without Xn interface

#### 5.1.4.x.1	Introduction

This solution proposes the handover procedure for Inter AMF, Intra SMF, Inter NG RAN (without Xn interface) scenario.

#### 5.1.4.x.2	Solution details

#### Figure 5.1.4.x.2-1 shows the procedure for Inter AMF, Intra SMF, Inter NG RAN handover without Xn interface.

The procedure for Inter AMF, Intra SMF, Inter NG RAN handover without Xn interface is as follows.

- If the handover is required, the UE sends the Measurement Report to the source NG (R)AN.

- The source NG (R)AN evaluates the report and makes the handover decision.

- The source NG (R)AN sends the Handover Required (UE Security Capabilities, Handover Restriction List, NSSAI) command to the source AMF.

- The source AMF sends the Forward Relocation Request to the target AMF.

- The target AMF sends the Handover Request (UE Security Capabilities, Handover Restriction List, NSSAI) to the target NG (R)AN.

- The target NG (R)AN checks if the NSSAI can be supported or not.

- If the UE qualifies for accessing the slice via the new NG (R)AN, the target NG (R)AN refreshes the AN keys (derives KAN* and the subsequent RRC and UP integrity and encryption keys).

- It then sends the Handover Request ACK to the target AMF.

- The target AMF derives the KNAS-MM*.

Figure 5.1.4.x.2-1: Inter AMF, Intra SMF, Inter NG RAN handover without Xn interface.

- The target AMF sends the Forward Relocation Response to the source AMF.

- The AMF sends the Handover Command to the source NG (R)AN.

- The source NG (R)AN sends the Handover Command to the UE.

- Using the parameter sent in the Handover Command, the UE derives the KNAS-MM*, KAN* and the subsequent RRC and UP integrity and encryption keys.

14. The UE sends the Handover Complete command to the target NG (R)AN.

#### 5.1.4.x.3	Evaluation

#### FFS

***** End of Change *****