# 3GPP TSG SA WG3 (Security) Meeting #86Bis	S3-170900

# 27 – 31 March, 2017, Busan, Korea	revision of S3-170709

# Source:	Huawei, Hisilicon, China Mobile

# Title:	Proposed agreement for the choice of non-AKA Authentication Method

# Document for:	Approval, Information, Discussion

Agenda Item:	5.1.2

# 1	Decision/action requested

In this document, we propose that EAP-TLS shall be standardized as non-AKA authentication method in Phase 1. Since Identity-based Signature (IBS) specified in RFC 6507 can further optimize the performance of EAP-TLS, we propose that IBS shall be supported by EAP-TLS but is optional to use.

# 2	References

# 3	Rationale

- SA3 has studied various non-AKA authentication methods based on EAP authentication framework in section 2.8, 2.9 and 2.14. EAP-TLS, EAP-TTLS and EAP-PSK have been studied in these solutions.

- EAP-TLS an authentication is a mature authentication protocol and is widely used for mutual authentication over Internet. Therefore, we propose that EAP-TLS shall be standardized in the phase 1 for non-AKA authentication.

- Normally, EAP-TLS uses certificate for mutual authentication. However, for some computing constraint devices, public key technology such as IBS specified in RFC 6507 is more efficient in both computing and transmission. Furthermore, with IBS public/private key technology, the operator can achieve the same level of control over credential as symmetric key. The credential can be done in an offline manner and thus can further reduce cost. Therefore, we propose that using IBS with EAP-TLS shall be standardized in pahse 1 also.

# 4	Detailed proposal

It is proposed to approve below pCR.

**********************Begin of changes********************************

### E.2.1 Questions and Agreements for Key Issue #2.1 Authentication Framework

#### E.2.1.0 Questions in other clauses affecting this key issue

tba

#### E.2.1.1 Support for AKA and its variants for primary authentication

##### E.2.1.1.1 Description of Question

Question: shall the 5G network support a variant of the AKA authentication protocol for primary authentication? If so, which variant or variants shall be supported? Variants of the AKA protocol under discussion include EPS AKA, EPS AKA*, EAP-AKA, EAP-AKA’, EAP-AKA*.

In answering this question, the aspects of serving network authentication, efficiency, and increased home control shall be explicitly taken into account.

##### E.2.1.1.2 Agreement

tba

#### E.2.1.2 Support for EAP framework for primary authentication

##### E.2.1.2.1 Description of Question

Several solutions propose support for the EAP framework for primary authentication. This implies that the access network can carry EAP messages.

Question:

Shall the 5G network support the EAP framework for primary authentication in the above sense?

If so, shall other authentication methods than EAP methods be allowed? The answer should be given separately for 3GPP and non-3GPP accesses.

##### E.2.1.2.2 Agreement

#### tba

#### E.2.1.3 The choise of EAP methods

##### E.2.1.3.1 Description of Question

Several solutions propose support for various EAP methods, e.g. EAP AKA, EAP AKA’, EAP TLS, EAP TTLS, and EAP PSK.

Question:

Exactly which EAP methods shall be standardized for 5G access in 3GPP?

##### E.2.1.3.2 Agreement

EAP-TLS shall be  standardized for 5G access in 3GPP at least but is optional to use.

Identity-based signature in EAP-TLS shall be standardized in 3GPP standard but is optional to use.

*********************************End of changes*******************************