# 3GPP TSG SA WG3 (Security) Meeting #89	S3-173218

# 27 November – 1 December, 2017, Reno, USA	revision of S3-17xabc

# Source:	China Unicom

# Title:	Security visibility

# Document for:	Approval

Agenda Item:	7.2.7 Visibility & configurability

1	Introduction

This contribution proposes to add security reuqirenment for TS33.501 clasue 5.4.1 security visbility.

Next generation system is expected to diverse access networks (Section 4.1 of TR 23.799), services (Section 5 of TR 22.891), and UE types. Different networks or services may have different security capabilities, in order to improve the user experience on awareness of security, it is proposed that add that the detail information including security capability or parameters should present for the advanced users’ reference in TS 33.501.

We kindly propose SA3 to agree the following pCR.

2	Proposal

It is proposed to approve below pCR.

**********************Begin of changes********************************

5.4.1	Security visibility

Editor's Note: It is FFS to clarify, if necessary, what "indication" means in this TS, i.e., data accessible via some interface/API or the actual display. Because, it is 3GPP TS 22.101 that describes how to use the "indication".

Although in general the security features should be transparent to the user or application, for certain events and according to the user's or application's concern, greater visibility of the operation of following security feature shall be provided:

-	Indication of access network encryption: the property that the user or application is informed whether the confidentiality of user data is protected on the radio access link.

The UE shall indicate to the user or application that the access network encryption is not switched on if any of the data radio bearers of the UE uses a null encryption algorithm.

The UE should indicate to the user that the detail information including security capability or parameters is present or not.

The ciphering indicator feature is specified in 3GPP TS 22.101 [20].

Editor's Note: It is FFS whether any additional security indicators are needed.

*********************************End of changes*******************************